February 9, 2009

To:	Federal Communications Commission

Public Safety and Homeland Security Bureau

From:	Region 24 800 MHz NPSPAC Regional Planning

Committee Chairperson Stephen T. Devine

Topic:	Region 24 800 MHz plan amendment, PR Docket 93-131

Dear Sirs:

We submit for consideration a plan amendment to the existing Region 24 800 MHz NPSPAC plan to modify channel allotments in existing counties as well as update outdated technical parameters that are inherent in the original Region 24 plan that can contribute to improved spectrum implementations and efficiencies within Region 24.

Background

Region 24 consists of the entire State of Missouri with its 114 counties plus the City of St Louis.

The Region 24 800 MHz Regional Planning Committee was established in 1989 at a meeting convened in Jefferson City, Missouri with 81 attendees.  An 800 MHz plan was subsequently developed and coordinated with Region 24’s eight (8) adjacent regions and accepted by the Federal Communications Commission on July 12, 1993.

Since the original plan was put into place, approximately 6 public safety radio systems utilizing NPSPAC 800 MHZ allocations from this plan have been licensed by the FCC, mostly in Missouri’s higher population areas of St Louis, Kansas City, Joplin and St Joseph.  Recent public safety communications initiatives in the St Louis area have found the existing 800 MHz channel allotments lacking due to several factors.  Those factors are:

Few channel allotments were allotted to Region 24’s St Louis area (specifically, the counties of St Louis, St Charles, Jefferson and Franklin along with the independent City of St Louis which is not located in any Missouri county) due to Region 24’s late filing of its plan with respect to its neighboring regions.  The late filing left few channels available to applicants in the St Louis Metropolitan area that could stand the scrutiny of adjacent region approval.

Over the years, 800 MHz system development has not flourished in the St Louis Metropolitan area for a number of reasons, with one of those reasons being the lack of 800 MHz channel availability.  Other issues including funding and overall regional governance contributed to the lack of regional 800 MHZ deployment.  This document and Region 24 800 MHz plan amendment is intended to alter the Region’s 800 MHz NPSPAC plan to make available the 800 MHz resources that have been lacking previously in the St Louis area with improved engineering tools and incorporating new heuristics into the plan to accommodate the engineering associated with the channel availability that results from the implementation of new digital 800 MHz public safety Land Mobile Radio (LMR) implementations.

Thank you for reviewing the information we have brought forth in this 800 MHz plan amendment.  The portions of the plan that have been modified are highlighted in Yellow text to show how the proposed plan diverts from the original language.  We feel it is a good practice to include in our filing a copy of the current Region 24 800 plan that is on file to easily show the areas that have been updated and to allow for a comparison between the current and proposed plan.

In general, the areas that have received the most alteration are the addition of Appendix K, which is a side by side comparison of the old 800 MHz NPSPAC channel numbers and their corresponding frequencies with the new, anticipated channel numbers and frequencies that will result from 800 MHz re-banding and Appendix L which is a new list of channel allotments for Region 24’s 114 counties and the City of St Louis.  Channel allotments that have been altered from the current plan are highlighted in Yellow to highlight the new additions and allow for easy comparison to the existing plan.

To maximize 800 MHz channel availability in the St Louis area amongst the counties of St Louis, St Charles, Jefferson and Franklin as well as, to a limited degree, the City of St Louis, a “zoned approach” for using channel allotments will be utilized.  This approach will allow the transfer of channel allotments between counties within the same zone.  The zones are established as:

North Zone	St Louis, St Charles Counties and the City of St Louis

South Zone	Franklin and Jefferson Counties.

Utilizing a “zoned” approach when distributing channel allotments can leverage a greater amount of area to implement systems and improve adjacent channel use in conjunction with existing systems.  In addition to the approval for new channel allotments in the St Louis area, any channel allotments utilized in NPSPAC applications will require an additional adjacent region concurrence after the initial allotment is approved.

Other changes to the Region 24 800 MHz plan have been made outside of additional channels allotted within the St Louis area.  They include:

Additions of channel allotments to seven (7) counties in Mid-Missouri.  These counties are the only counties in Missouri more than seventy (70) miles from an adjacent region and, since channels are able to be added to them without impact to any adjacent region, some channels were added to better manage the available 800 MHz channels in these areas.

Alteration of channel allotments in these areas is possible as these counties are more than 70 miles from any adjacent region.  These counties are:

Miller

Cole

Moniteau

Pulaski

Cooper

Camden

NSPAC Region 24 Missouri 800 MHz Plan

Chairman John R. Gerke

City of St. Louis Communications Division

4971 Oakland Ave.

St.  Louis, MO.  63110    314-533-5802

ITEMS TO CHECK PERTAINING TO THE PLAN OF REGION 24, MISSOURI

1)   Cover page - identifying the region.

2)   Chairperson - name, address, phone number, and signature.

See page 19 and under cover page.

3)   Committee members - name, organizational affiliation, address, phone

numbers.  See Appendix H.

4)   Summary of major elements of the plan.  See page 1.

5)   General description of how spectrum is allotted among users.  See page 13.

6)   Explanation of how the requirements of all eligibles are considered and met.

See page 18.

7)   Explanation of how eligibles are prioritized in areas where not all eligibles

may receive licenses.  See page 18.

8)   Explanation of how the plan has been coordinated with adjacent regions.

See page 12 and appendix J.

9)   Description of how the plan puts spectrum to best possible use by:

I.   requiring system design with minimum coverage areas See page 10.

II.   assigning frequencies so that maximum frequency reuse and

offset channel use may be made.  See page 13.

III.   making use of trunking.  See pages 9 and 16.

IV.   requiring small entities with minimal requirements to join

together on a single system where possible.  See page 10.

10)  Explanation of how interoperability channels are managed.  See page 8.

11)  “Slow Growth” language.  See page 17.

12)  Does the plan refer to Give-Back frequencies? If yes, give page number 12.

13)  Use the APCO sorting program.  See page 13.

14.)  Appeal process.  See page 19.

15)  Does the plan provide for regional mutual aid channels in addition to the five

(5) common channels? If so, are their guard bands for these channels.  (NO)

16.   Similar to the Generic Plan describe the formation of the committee:

I.    Advertising - copy should be attached to legal notice,

letters to the industry, etc.

II.    Who could vote and what procedure was used after the

first meeting.  See page 6.

III.   How was the final plan adopted? Was in by members

attending a meeting or mail ballot? (BALLOT)

TABLE OF CONTENTS

1.0  SCOPE

1.1  Introduction................................................................................................................................	6

1.2  Purpose.......................................................................................................................................	6

2.0 AUTHORITY

2.1     Regional Planning Committee.................................................................................................	6

2.2     Planning Committee Formation..............................................................................................	7

2.3     National Interrelationships......................................................................................................	7

2.4     Federal Interoperability..........................................................................................................	8

2.5     Regional Review Committee..................................................................................................	8

3.0 SPECTRUM UTILIZATION

3.1     Region Defined.......................................................................................................................	8

3.2     Region Profile (Demographic Information)............................................................................	8

3.2.1              Geographical Description............................................................................................9

3.2.2              Missouri Population....................................................................................................	9

3.3     Usage Guidelines....................................................................................................................	9

3.4     Technical Design Requirements for Licensing.......................................................................	10

3.4.1               Definition of Coverage Area.....................................................................................	10

3.4.2               System Coverage Limitations....................................................................................	10

3.4.3               Determination of Coverage........................................................................................	10

3.4.4               Annexation and Other Expansions.............................................................................	11

3.4.5               Coverage Area Description.......................................................................................	11

3.4.6               Give-Back Frequencies.............................................................................................	12

3.4.7               Reserved Spectrum....................................................................................................	12

3.4.8               Adjacent Region Considerations...............................................................................	12

3.5      Initial Spectrum Allocation...................................................................................................	13

3.5.1                Frequency Sorting Methodology...............................................................................	13

3.5.2                Geographic Area.......................................................................................................	13

3.5.3                Blocked Channels......................................................................................................	13

3.5.4                Transmitter Combining.............................................................................................	13

3.5.5                Special Considerations.............................................................................................	13

3.5.6                Protection Ratios......................................................................................................	14

3.5.7                Adjacent Region Coordination..................................................................................	14

4.0 COMMUNICATIONS REQUIREMENTS

4.1      Common Channel Implementation........................................................................................	14

4.1.1                 Areas of Operation..................................................................................................	14

4.1.2                 Operation on the Common Channels.......................................................................	14

4.1.3                 Operation Procedures.............................................................................................	15

4.1.3(I)                            International Calling Channel (ICALL)....................................................	15

4.1.3(II)                           International Tactical Channels (ITAC1 thru ITAC4)..............................	15

4.1.4                 Coded Squelch........................................................................................................	15

4.2      Network Operating Method...................................................................................................	15

4.3      Requirements for Trunking....................................................................................................	16

4.4      Channel Loading Requirements.............................................................................................	16

4.4.1                 Loading Tables.......................................................................................................	17

4.4.2                 Traffic Loading Study..............................................................................................	17

4.4.3                 Slow Growth............................................................................................................	17

4.5      Use of Long Range Communications....................................................................................	17

4.6      Expansion of Existing Systems..............................................................................................	17

5.0 IMPLEMENTATION AND PROCEDURES

5.1      Notification...........................................................................................................................	18

5.2      Frequency Allocation Process................................................................................................	18

5.3      Frequency Allocation Maps...................................................................................................	18

5.4      State Map................................................................................................................................	18

5.5      Expansion of Initial Allocation..............................................................................................	18

5.6      Prioritization of Applicants....................................................................................................	18

5.7      Appeal Process......................................................................................................................	19

6.0 THE REGIONAL PLANNING COMMITTEE..........................................................................	19

1.0  SCOPE

1.1  Introduction

In December of 1983, the United State congress directed the Federal Communications Commission (FCC) to establish a plan to ensure that the communications needs of state and local public safety authorities would be met.  By their regular means of initiation, the FCC began the process of developing such a plan.  Through their efforts, and the efforts of the National Public Safety Planning Advisory Committee (NPSPAC) the plan was begun.

The National Public Safety Planning Advisory Committee provided an opportunity for the public safety community and other interested members of the public to participate in an overall spectrum management approach by recommending policy guidelines, technical standards, and procedures to satisfy public safety needs for the foreseeable future.  After consideration of NPSPAC’s Final Report and comments filed in the Docket No.  87-112, a Report and Order was released by the FCC in December 1987, which established a structure for the National Plan that consists of guidelines for the development of regional plans.

The National Plan provides guidelines for the development of regional plans.  The particulars of the plan are found in FCC 87-359, which contains the required steps and contents for regional plan development.  It is on this document that this plan is developed.

1.2  Purpose

Public safety communications has, for many years, been inadequate throughout the United States.  This is as true for Missouri as it is for any other state.  Many, if not all, public safety radio users are constantly bombarded with outside interference, noise, and over-crowding.  It is with these problems in mind that this  plan was developed.

This regional plan was developed with the objective of assuring all levels of public safety/public service agencies that radio communications in the near and distant future will not suffer from the problems of the past.  The allocation of frequencies was done in as equitable a way as possible.  The goal was to supply a pool of frequencies for each county and a pool for state agency use with adequate reserve allocations for future needs in all areas, and a method to appeal initial allocations based on need.

The National Plan, as developed by NPSPAC, was followed very closely in all considerations for frequency allocation, re-use, turn back, regional interoperability, spectrum requirements, and adjacent region operations.  This plan should provide the flexibility to accommodate the growth and changes which are bound to occur in public safety and public service communications operations long into the future.

2.0  AUTHORITY

2.1  Regional Planning Committee

The development of the Public Safety Radio Communications Plan for Region 24, the State of Missouri, has followed the requirements of the FCC’s Report and Order as issued in the matter of General Docket 87-112.

In accordance with the FCC’s Report and Order 87-112, the  Associated Public Safety Communications Officers, Inc, (APCO) recommended to the Commission the appointment of a “Convener” for Missouri Region 24.  The convener served as the coordinator for the assembly and formation of the planning committee.

Participants in the formation of the Regional Planning Committee represent interested parties from both the Public Safety and Special Emergency Radio Services.  A total of 81 individuals convened to participate in the development process.  The list herein contains the names, organizational affiliations, and phone numbers of all individuals that convened.  (Appendix A)

The committee was selected by attendance at the planning meetings.  Each member of the Committee representing an eligible licensee under the Public Safety Radio Services and the Special Emergency Radio Services was entitled to one vote in all Committee matters.  Except as may be provided elsewhere in the Plan, the majority of those present at a scheduled meeting constituted a majority for all business.  Only the final approval of the plan prior to submission to the FCC required a vote from more than would be in attendance at a regular meeting.  In this case the vote was conducted by mail ballot sent to all those who had participated in the planning process.  This way, the finished plan was reviewed and accepted by the widest, within reason, group of Public Safety/Public Service users.

2.2  Planning Committee Formation

The process of forming the Planning Committee was conducted in the following steps:

Presentations concerning the requirements for a regional planning committee were presented and discussed at state organization meetings.  At each presentation there was an opportunity for persons to place themselves and/or their agency on the mailing list.

Letters of announcement were mailed to each major state agency radio user, those placed on the mailing list, as well as to state organizations composed of local government level public safety/public service users.  Letters were also sent to all members of the Missouri Chapter of APCO.

This first meeting was held at the Missouri State Highway Patrol Headquarters in Jefferson City, Missouri - a public facility.  The Chairperson, as well as a secretary, waselected at this meeting.  An interim committee of 33 persons (Appendix B) was created to conduct business until the planning committee was established.  Committee membership was left open to any person or agency which may or may not have been notified or decided to join the committee later.

The planning committee of 18 persons (Appendix C), including the Chairperson and the secretary was established at a second meeting held at the same site.  This meeting was attended by 22 of the 33 interim committee members (Appendix D).

Both the original Chairperson and the original Secretary stepped down at later dates.  A third meeting was called at the same site, attended by 17 persons, (Appendix E).  A new Chairperson was elected at this meeting.  Later, in compliance with the provisions of this plan , 5 additional persons were admitted to the committee (Appendix F).  The Planning Committee, by this time, was comprised of 23 members, including the Chairperson (Appendix G).  On December 2, 1992 these 22 members were solicited by means of certified mail to ascertain their intent to remain as bonafide members of the planning committee.  The result is that the Committee of Record is comprised of 18 members, new and old (Appendix H).

Vendor participation was encouraged, but vendors were not allowed a vote.

2.3  National Interrelationships

The Regional Plan is in conformity with the National Plan.  If there is a conflict between the two plans, the National Plan will govern.  It is expected that Regional Plans for other areas of the country may differ from this plan due to the broad differences in circumstances, geography, and population density.  By officially sanctioning this plan the Federal Communications Commission agrees to it’s conformity to the National Plan.  Nothing in the plan is to interfere with the proper functions and duties of the organizations appointed by the FCC for frequency coordination in the Private Land Mobile Radio Services, but rather it provides procedures that are the consensus of the Public safety Radio Services and Special Emergency Radio Service user agencies in this Region.  If there is a perceived conflict then the judgment of the FCC will prevail.

2.4  Federal Interoperability

Interoperability between the Federal, State, and Local governments during both daily and disaster operations will primarily take place on the five common channels identified in the National Plan.  Additionally, through the use of S-160 or equivalent agreements, a licensee may permit Federal use of a non-Federal communications system.  Such use, on other than the five identified common channels, is to be in full compliance with FCC requirements for government use of non-government frequencies (Title 47 CFR, sec 2.103).  It is permissible  for a non-Federal government licensee to increase channel requirements to account for 2-10 percent increase in mobile units, dependent on the amount of Federal Government Agencies involvement in it’s area, provided that written documentation from Federal agencies supports at least that number of increased units.

2.5  Regional Review Committees

Upon approval of this Plan by the Federal Communications Commission, a Region Review Committee will be established for the review of applications which do not fall within the stated guidelines provided for in this plan, or for the settlement of disputes concerning this plan and/or its application.

This committee shall consist of the Local APCO Frequency Advisor for this region, one representative from the Police, Fire, and EMS services, and a minimum representation from other eligibles is also welcome.  This committee and its composition will be assured by the Missouri APCO chapter and other Public Safety organizations.  Membership on this committee will be solicited on an annual basis.  Since this committee will probably not have regular business, it will be up to the Local APCO Frequency Advisor to notify the committee of problems, conflicts, or when it becomes apparent that spectrum demands will outpace available spectrum.  Each member of the committee shall be furnished a copy of this plan upon their appointment or election to the committee.

This committee will serve as a referee to resolve disputes, arbitrate in matters involving spectrum availability versus spectrum for which applications have been filed, and will be the final step in contested applications within Missouri Region 24.  This committee will be a standing, open committee, in which each service or discipline will have one vote.

Plan updates shall be accomplished by this committee.  All changes or updates to the plan shall be first agreed upon by this committee and then submitted to the FCC for their review and consideration.  When approved all changes shall be added to the plan with the appropriate documentation of approval.

This committee shall meet at least once annually to review the implementation of the plan.  This review shall consist of examination of any and all license activity.

3.0  SPECTRUM UTILIZATION

This portion of the Plan provides a basis for proper spectrum utilization.  Its purpose is to guide the Local APCO Frequency Advisor and/or the Regional Review Committee in their task of evaluating the implementation of this plan within the Region.

3.1  Region Defined

Region 24 is the State of Missouri.  This region is the result of definition by the Federal Communications Commission as a result of recommendations made in the National Public Safety Planning Advisory Committee (NPSPAC) plan as submitted and approved and contained in Docket 87-112.  For purposes of this plan Region 24 shall be defined as all the lands and waters contained within the boundaries of the State of Missouri.

3.2  Region Profile (Demographic Information)

The purpose of this section is to provide the basis for the assignment of frequencies, and their re-use.  Since the frequency allocation formula used is based on population within a county, it is necessary to provide this information within this plan.  The below dates used in the determination of frequency allocations.

3.2.1  Geographical Description

There are 115 counties in the State of Missouri.  This includes the City of St. Louis that is a county by statute.  Therefore St.  Louis County and the City of St.  Louis are independent counties.

The total land mass of the state is 69,697 square miles.  The largest county is Texas County, with a total of 1180 square miles.

3.2.2  State of Missouri Population and Expected Growth Percentage

The population of the state (1990 census) is 5,117,073.  Some 60% of the populace resides in 14 of the 115 counties.  Based on the Missouri State Demographer’s calculations, the total population for the state is expected to increase to 5,453,441 (1.1%) by the year 2010.  (See Appendix I for Census & Demographer’s data).

The population per square mile (1990 & projected) is somewhat sparse in some areas of the state.  Worth County, in the hear 2010, is predicted to have 6.5 persons per square mile.  St.  Louis County is predicted to have 2050 persons per square mile.  This presents some problems in area coverage for radio systems in that the entire land area of any given jurisdiction must be covered.  All of these conditions may have been taken into consideration in the allocation plan.

3.3  Usage Guidelines

All systems operating within the Region having five or more channels will be required to be trunked.  Those systems having four or less channels may be conventional or trunked.

The FCC, in its Report and Order states, “Exceptions will be permitted only when a substantial showing is made that alternative technology would be at least as efficient as trunking or that trunking would not meet operational requirements.  Exceptions will not be granted routinely, however, and strong evidence showing why trunking is unacceptable must be presented in support of any request for exception”.

Systems of four or less channels operating in the conventional mode who do not meet FCC loading standards will be required to share the frequency on a non-exclusive basis.

Public Safety communications at the state level, as it impacts the Region, will be reviewed by the Committee.  State-wide public safety agencies will submit their communications plans for impact approval if they utilize communications systems within the Region and those portions of such systems must be compatible with the Regional Plan.

The next level of communication coverage will be a county/multiple municipality area.  Those systems that are designed to provide area communication coverage must demonstrate their need to require such wide area coverage.

This would apply in a situation such as a city requesting coverage of an entire county.  Communication coverage beyond the bounds of a jurisdictional area of concern cannot be tolerated unless it is critical to the protection of life and property.  If the 800 MHz trunked radio technology is utilized, the system design must include as many county/multiple municipality government public safety and public service radio users as can be managed technically.

The county/multiple municipality agency(ies), depending upon systems loading and the need for multiple systems within an area, must provide intercommunications between area-wide systems.  In a multi-agency environment, a lead agency, using the 800 MHz spectrum, which is an agency or organization having primary response obligations in the geographic area, shall be responsible for coordinating the implementation of the Common Channels in this band as mandated by the National Plan.  Such implementation must be reviewed and approved by the Local APCO Frequency Advisor, and at his/her discretion, the Regional Review Committee.

Municipal terminology often differs.  In order to provide a title for the next levels of communications the term ‘Municipality’ will be used to define the level below county-wide.  ‘Municipal’ communications for public safety and public services purposes must provide only the communications needed within its boundaries.  However, if the total number of radios in service does not reach minimum loading criteria for a trunked system, that City or Township must consider utilizing the next higher system level if 800 MHz trunked radio is available in the area.  As those higher level systems reach capacity, the smaller system communicators in public safety and public service must then consider uniting their communications efforts to formulate one large system or forfeit use of the limited 800 MHz spectrum.

Where smaller conventional 800 MHz needs are requested, those frequencies to be utilized must not interfere with the region’s trunked system. The 800 MHz trunked radio system is to be considered the higher technology at this time and in greater compliance with FCC guidelines.  The amount of interference that can be tolerated depends on the service affected.  Personal life and property protection shall receive the highest priority and disruptive interference with communications involved in these services in an area shall not be tolerated.  Any co-channel interference within an authorized area of coverage will be examined on a case-by-case basis by the Regional Review Committee.

3.4  Technical Design Requirements for Licensing

3.4.1  Definition of Coverage Area or Area of Jurisdiction

The coverage area shall be that area for which a system is intended to cover with a received signal strength of greater than 40 dBu. This area shall normally represent the boundaries of the County or the incorporated municipality which is applying for the license.  In the case of regional or area-wide, multi-jurisdictional systems, the coverage shall be that area of all jurisdictions participating in the system combined.

3.4.2  System Coverage Limitations

System coverage shall be limited to the coverage area defined as listed above plus no more than five (5) additional miles in all directions extending from said boundaries of definition.  This limitation shall assure maximum frequency reuse.  The only exception to this rule shall be those applicants wishing to offer service or system use to areas outside of their jurisdictional boundaries.  In these situations the applicant shall provide a proposal of said service to the Local APCO Frequency Advisor, who may request Regional Review Committee consideration for approval.

Systems not located within the geographical center of the jurisdiction(s) for which they cover shall utilize either directional antennas or antenna/tower relationship techniques to achieve the coverage required by this plan.

3.4.3  Determination of Coverage

There are four variables used in determining the area of coverage of a proposed system.  These variables are (1) the required strength of the received signal (2) antenna height above average terrain (HAAT), (3) the effective radiated power (ERP) of the system, and (4) the type of environment.

Received signal strength:

For purposes of this plan, received signal strength shall be the determining factor which defines the actual boundary of a system.  The signal level which marks the outer boundary of a system shall be 40 dBu.

Antenna Height:

Shall be the height of the antenna above the average terrain surrounding the tower site.

Effective Radiated Power:

The ERP is the transmitter output power times the net gain of the antenna system.  The actual formula is: ERP (W) equals Power (w) times Antilog (net gain in db divided by 10).

Environment Type:

The following four classifications were used to describe average terrain:

1-URBAN:  Which is a built-up city crowded with large buildings or closely interspersed with houses and thickly grown trees.  This would include the downtown area of a major city.

2-SUBURBAN:  Which is a city scattered with trees, houses, and buildings.  This would include the downtown area of a large city.

3-QUASI-OPEN:  Is an area between suburban and open areas.  This includes areas outside of city limits that have few buildings and houses.

4-OPEN:  Is an area where there are no obstacles such as tall trees or buildings in the propagation path or a plot of land which is cleared of anything for 300 to 400 meters ahead.  This would include farm land, open fields, etc.

3.4.4  Annexations and Other Expansions

It is well known that, as cities grow, annexations occur.  When an expansion of the present city limits of any city currently using an 800 megahertz system within the spectrum as herein specified occurs, it is understood that the existing system may have to be expanded and its range increased.  This is a modification and may be permitted.  The increased range of the system will have to be determined at the time of modification to assure non-interference with any other existing systems.  Where interference is likely, the use of alternate methods of expansion, such as satellite systems, may be necessary.  Where more spectrum is not available from the initial allocation, the rules for expansion of initial allocation, as contained in this plan, shall apply.

3.4.5  Coverage Area Description

All applicants shall provide with their applications a map showing the jurisdictional boundaries to be covered by the page system, and the calculated system coverage.  This map shall display the location of the system transmitter(s), including control stations.  It is recommended that a U.S. Geological Survey (USGS)

Quad topographical map be used for this purpose.  If not available, a high quality locally produced map or a highway map may be substituted.  Regardless of the type map used, the name of the applicant and the scale of the map shall be displayed on the map.

3.4.6  Give Back Frequencies

All agencies participating in the use of the new 800 megahertz spectrum shall prepare and submit a plan for their currently licensed frequencies in the lower bands.

Agencies will be required to surrender their vacated frequencies if the Region 24 800 MHz regional planning committee agrees that the following conditions are met: (1) the new system fully replaces the functions of the old one, and (2) the new system has operated satisfactorily for a sufficient time period to permit a smooth transition from former operations and to demonstrate the degree of reliability necessary for the applicant to complete their mission.  (refer to NPSPAC Final Report, Section V-A).  We anticipate applicants initially providing, in their applictions to the Region 24, a general idea of the number of frequencies (above and below 512 MHz) that will be returned to the availability pool.  The degree of “give back” spectrum returned to the region will be a factor considered by the Region when reviewing applications for 800 MHz NPSPAC channels.

Entities within a region will have the freedom to consider below 800 MHz public safety bands in developing their regional plans, but the licensing of channels in the bands would continue to be conducted through existing frequency coordination procedures.

Frequencies which are to be abandoned by an agency shall not be handed down to another agency within the respective jurisdiction.  Though this may seem a convenient method to re-use existing radio equipment, the reassignment must be handled through the normal process.  It is recommended that any jurisdiction wishing to “hand down” frequencies to another agency submit the proper coordination and application forms with the document of release.  This will put the applicant in a better posture for reassignment of the frequency in question.  It should be noted that even though this procedure is followed, there is no guarantee that a particular frequency will be assigned to the requesting jurisdiction.

The time frame allowed for phasing into 800 MHz and out of the lower currently licensed bands will be considered on a case-by-case basis by the regional planning committee.  Generally, one year will be considered acceptable in most cases, with 18 months as a maximum.  Any agency requiring more than 18 months shall provide documents stating the reason for the delay, and give the estimated time of completion.

Regional 800 MHz system deployments in which the applicant is a governing body that will provide services to many existing user agencies and licensees must coordinate the return of the channels to the public safety pool for the agencies and users the represent on their application.  For example, if a system is applied for to the Region 24 800 MHz RPC with the intent to provide service to all law enforcement agencies in a particular county but the applicant is not an existing licensee(s), it will be the responsibility of the applicant to coordinate the transition of the VHF/UHF channels licensed to those they intend to provide service to in the application to the general public safety pool.

3.4.7  Reserved Spectrum.

Due to the fact that all of the frequency spectrum is not needed at this time, the excess channel pairs will be returned to a reserve spectrum.  These channels may be used for conflict with adjacent Region allocations or may simply remain within the Region until needed.  This does not imply that these frequencies are unavailable, only that before they can be utilized within the Region they must be coordinated via the regular APCO coordination process and within the guidelines set forth in this plan.  Where possible, the channels designated for a jurisdiction in this plan shall be used.

3.4.8  Adjacent Region Coordination

Coordination with adjacent regions shall be an on-going process.  There are eight Regions Adjacent to Region 24, State of Missouri: Arkansas, Region 4; Illinois, Region 13; Iowa, Region 15; Kansas, Region 16; Kentucky, Region 17; Nebraska, Region 26; Oklahoma, Region 34; and Tennessee, Region 39.  All of these Regions were sent copies of the Region 24 plan by certified mail.  Refer to appendix J for responses.

As the use of the five National channels is not considered a day-to-day function, the “hard” coordination for the use of these channels is not considered to be necessary or advisable.  The use of these channels will always be on a non-interference basis, with on-the-air coordination at the time of use when required.  Any user found to be operating in any manner other than this shall be considered to be operating improperly and subject to the existing Federal Communications Commission rules for willful interference with the communications of other users.

3.5  Initial Spectrum Coordination

3.5.1  Frequency Sorting Methodology

The initial spectrum allocation for the Region was determined by a computerized frequency sorting process performed by APCO.  The purpose of the computer program which assigns frequencies to specific eligibles and to pools for future assignments is two-fold:

A)  The assignments must result in a high degree of spectrum efficiency.

B)  The assignments must result in a low probability of co-channel and adjacent channel interference.

Since the desired output is a geographic sorting of frequencies, a method of defining geography must be part of the input.  A list of the number of channels to be assigned in each geographic area is also required.

Acceptable interference probabilities are determined for the Region.  Frequency assignments are then made using a computer program which satisfies the goals of spectrum efficiency and interference protection.  The following narrative describes the factors and process used by the computer program.

3.5.2  Geographic Area

For the purpose of this frequency sort, a geographic area is defined as one or more circles of equal radius.  To the degree practical, the circle(s) should include the entire area of the eligibles geopolitical boundary, but not exceed the boundary by more than five (5) miles.  Thus, the procedure is to gather maps of sufficient detail, outline the areas to be defined, determine the coordinates and radius of the circles which define each area, and tabulate the data.

3.5.3  Blocked Channels

In the Region there are five mutual aid channels which must be blocked out to prevent the computer from making assignments on these channels.  (Since the mutual aid channels are spaced at 0.5 MHz intervals, other Region-wide systems are spaced at 0.5 MHz and placed adjacent to the mutual aid channels.  This procedure reduces the impact of blocked and adjacent channels by virtue of the fact that the channel plan already has protection spacing on each side of the mutual aid channels.)

These Region-wide blocked channels are identified by FCC channel number, tabulated and they become input to the computer program.

3.5.4  Transmitter Combining

The computer program is designed to provide a minimum frequency separation between any two channels assigned to the same eligible at the same site.  This separation is provided in order to enable more efficient combining of multiple transmitters to a single antenna.  These separated blocks of frequencies also have a maximum size.  That is, if the eligible has more frequencies than the maximum size of the combining block, then a second compatible block is created, and so on.  Each of these parameters is adjustable in the program on a global basis.  The default parameters chosen are 0.25 MHz minimum spacing and five channel blocks.

3.5.5  Special Considerations

There are licensees in the 806-821/852-866 MHz spectrum who plan to expand existing systems into the 821-824/866-869 MHz bands.  Some of the existing radio units are unable to operate on 12.5 KHz separated carries frequencies.  The result is that these radios can only operate on “even” FCC numbered channels in the 821-824/866-869 MHz band.  The computer program is able to take this into account when making assignments.

3.5.6  Protection Ratios

There are three interference protection ratios built in to the Region 24 allocation matrix.  One is for the co-channel case, the other two are for the adjacent channel case.  The co-channel ratio of 40/5 dBu provides 35 db Desired/Undesired signal ratio for the co-channel case.  The original NPSPAC allocations were based upon channels spaced at 12.5 kHz with an occupied bandwidth of +-20 kHz.  This resulted in bandwidth overlap between adjacent channels, and the required the need to geographically space adjacent channel allocations. The adjacent channel ratio of 40/25 dBu provides 15 dB Desired/Undesired signal ratio for the adjacent channel case. With the advent of narrowband operations at 800 MHz, particularly through the utilization of Project 25 interoperable standards, the adjacent channel bandwidth overlap has been eliminated.  The occupied bandwidth of a Project 25 transmitter is less than +-10 kHz and fits well within a channel spacing of 12.5 kHz.  As such, Region 24 will reduce the adjacent channel protection requirement for its 800 MHz NPSPAC Coordination to that established by the National Coordination Committee for 700 MHz at 40/60 dBu, where both then existing system and the proposed system are operating with bandwidths less than 11.25 kHz.  These ratios provide an acceptable probability of interference for Public Safety Service.

3.5.7  Adjacent Region Consideration

The computer program requires a listing of channels to be blocked along the borderline with other regions which have pre-existing plans.  If the adjacent region plan was developed using the APCO packing program, this information exists in the database.  If the adjacent region plan was developed by another method, then the data must be obtained from the adjacent region’s plan in order to build the exclusion list.

4.0  COMMUNICATIONS REQUIREMENTS

4.1  Common Channel Implementation

The implementation of the International Common Channels must follow the guidelines as set forth by the Federal Communications Commission by the approval of the National Plan.  These five common channels are accessible by all levels of government and shall be used in accordance with the provisions of the National Plan.  All mobile and portable equipment must be equipped to operate in the “talkaround mode” when required on the International Channels.

The International calling channel - channel 601 (821/866.0125 MHz) - shall be implemented as a full mobile relay.  Wide area coverage transmitters will be installed where applicable within a system.  Large system users (5 channels or more) of 800 MHz shall be required to monitor this channel at all times.  The area of coverage for this channel shall be equal to the area covered by the licensed system.  This may or may not require the use of satellite receivers within the area to meet this requirement.

The four International Tactical (ITAC) Channels will be assigned State-wide, for use as needed by all eligible licensees.  These channels are to be used in accordance with the National Plan and in compliance with the regulations as set forth by the Federal Communications Commission.  These channels require no special licensing, only that the users be eligible for licensing on the other Public Safety 800 MHz channels as specified in section 90.616 (a) of the FCC Rules and Regulations.

4.1.1  Areas of Operation

The common channels shall be available for use throughout the Region.  No specific assignments were deemed necessary within the Region.

4.1.2  Operation on the Common Channels

Normally, the five interoperable channels are to be used only for activities requiring inter-communications between agencies not sharing any other compatible communications system.  Interoperable channels are not to be used by any level agency for routine, daily operations.  In major emergency situations, one or more ITAC channels may be assigned by the primary Public Safety Agency within that area of operation.  The primary Public Safety agency in each county, if not defined elsewhere in the plan, shall be the County Sheriff’s Department or Public Safety Department or the lead agency, which may be any agency licensed to operate in this spectrum, or “on-scene” commander.  The primary Public Safety agency shall be the city level Public Safety Department in situations which occur within the corporate limits of said city.  These primary agencies will assign one or more of the ITAC channels for use according to need during each special situation requiring the use of these channels.

Participants in the interoperable channels include Federal, State, and Local Disaster Management agencies.  Police, Fire, and providers of Basic and Advanced Life support services will be the primary using agencies.  If radio channels are available, other Special Emergency Radio Services may also participate to the extent required to insure the safety of the public.  These agencies include the Highway Department, Motor Vehicle Comptroller, Forestry, Wildlife, and other special service agencies not normally involved in day-to-day public safety operations.

4.1.3  Operation Procedures

On all Common Channels, plain English will be used at all times, and the use of unfamiliar terms, phrases, or codes will not be allowed.

4.1.3(I)  International Calling Channel (ICALL)

The ICALL channel shall be used to establish contact with other users in a particular Region that can render assistance at an incident.  This channel shall not be utilized as an ongoing working channel.  Once contact has been established between agencies, and agreed upon ITAC or mutual aid channel shall be used for continued communications.

4.1.3(II)  International Tactical Channels (ITAC-1 - ITAC-4)

These frequencies are reserved for use by those agencies involved in inter-agency communications.  Incidents requiring multi-agency participation will utilize these frequencies as directed by the control agency assuming responsibility for an incident or area of concern.  These frequencies may be subdivided according to function in an incident or by geographical location in response to an incident.  It is recommended that the following assignments for ITAC-1 through ITAC-4 be used when possible:

ITAC-1.......................Law Enforcement

ITAC-2.......................Fire Services

ITAC-3.......................Emergency Medical Services

ITAC-4.......................Command and Control

4.1.4  Coded Squelch

All equipment capable of operating on the five (5) common channels shall be equipped with the National Common Tone Squelch of 156.7 Hz.  Mobile relays on these channels, if authorized, may use additional tone or digital squelch code for the purpose of selecting individual mobile relay stations, provided the National Common Tone Squelch Code is used on the output.  If such an arrangement is utilized, provision must also be made for certain centralized, high level sites to be activated by the 156.7 tone to ensure emergency access by transient units.

4.2  Network Operating Methods

Communications systems on ITAC-1 through ITAC-4 will be implemented by agencies who volunteer on a distributed coordinated basis.  Every primary geographic section of the Region is intended to be covered by at lest one of the ITAC channels.  In many areas the common channels will be utilized on a mobile to mobile talk-around basis.  Mobile relays in ITAC-1 through ITAC-4 will be on a limited coverage design to permit reuse of the channel several times within the Region and in adjacent regions.  Since Region 24 will probably not have a large number of stationary ITAC Channel stations, the implementation of mobile relay or repeaters is strongly encouraged.  This will fill an “on-scene” requirement for most multi-agency response situations.  Adjacent region coordination will be via existing mutual aid coordination procedures with the requesting region establishing the tactical frequency assignment.

4.3 Requirements for Trunking

All systems operating in the Region having five or more channels will be required to be trunked.  Those systems having four or less channels may be conventional.  It is strongly suggested that any entity licensing three or more repeaters use trunking.

The FCC in its Report and Order states: “Exceptions will be permitted only when a substantial showing is made that alternative technology would be at least as efficient as trunking or that trunking would not meet operational requirements.  Exceptions will not be granted routinely.  Strong showings as to why trunking is unacceptable must be presented in support of any request for exceptions”.

Systems that do not meet FCC loading standards can be required to share such frequencies on a non-exclusive basis.  Those agencies requesting Data channels only can be required to share channels with adjacent agencies wherever feasible or limit coverage to their geographic area.  Exceptions will be considered on a case-by-case basis by the Regional Review Committee.

Depending on systems loading and the need for multiple systems within an area, operators of wide area systems (including, but not limited to, designated “Monitoring Agencies”) must provide for coordination between area-wide systems and “Monitoring Agencies”.  Single municipalities or agencies must restrict design and implementation of their system(s) to provide only the communications needed within its geopolitical boundaries.  The use of trunked systems is encouraged.  However, if the total number of radios in service does not reach minimum loading criteria for a trunked system, that user must consider utilizing the next higher system level if 800 MHz trunked radio is available in the area.  As systems reach capacity, the smaller system users must consider consolidating their communications systems to formulate one large trunked system.

A requesting applicant for radio communications in the 800 MHz public safety services in the Region will be required to conform to the FCC loading criteria for its proposed system.  The provisions of this regional plan must be used as a guide for establishing any new systems.  Strict adherence for limiting the area of coverage to the boundaries of the applicant agency’s jurisdiction must be observed.  Overlap or extended coverage must be minimized, even where systems utilizing 800 MHz trunked radio systems are proposing to intermix systems for cooperative and/or mutual aid purposes.

Antenna heights are to be limited to provide only the necessary coverage for a system.  When antenna locations are restricted to only the “high-ground”, transmitter outputs and special antenna patterns must be employed to produce only the necessary coverage with the proper amount of ERP.  All necessary precautions are to be taken to gain maximum reuse of the limited 800 MHz spectrum.

4.4  Channel Loading Requirements

An agency/jurisdiction requesting a single frequency to replace a frequency currently in use that will be turned back for reassignment will not be required to meet loading requirements in order to obtain the new frequency.  However, if the single frequency is not loaded to more than 70 units within three years after the license is granted, the frequency will be available for assignment to other agencies on a shared basis in the event that other frequencies meeting the criteria for assignment are exhausted.  Shared use of a frequency is not interference free.  Users of single frequency systems may be required to provide the Regional Review Committee “confirmation of loading” for mobiles and portables as a method of validating system loading.  This exception shall apply to agencies having only one system and a single frequency.  Agencies/jurisdictions requesting multiple frequencies or employing trunking technology shall comply with the loading standards as outlined below or provide a “Traffic Loading Study” that meets the criteria as outlined below.

4.4.1  Loading Tables

EMERGENCY                                                                     NON-EMERGENCY

CHANNELS                   UNITS/CHANNEL                     CHANNELS                         UNITS/CHANNEL

1 - 5                                  70                                                1 - 6                                       80

6 - 10                                75                                                6 - 10                                     90

11 - 15                              80                                                11 - 15                                   105

16 - 20                              85                                                16 - 20                                   120

Agencies requesting additional frequencies must show loading of 100 percent or greater on their existing system.  Should a demand for frequencies exist after assignable frequencies become exhausted, any system having frequencies assigned under this plan four or more years previously and not loaded to at least 70 percent will lose operating authority on a sufficient number of frequencies to bring the system into compliance with the 70 percent loading standard.  Frequencies lost in this manner will be reallocated to other agencies to help satisfy the demand for additional frequencies.

4.4.2  Traffic Loading Study

Justification for adding frequencies, or retaining existing frequencies, can be provided by a traffic loading study in lieu of loading by number of transmitters per channel.  It will be the responsibility of the requesting agency to provide a verifiable study showing sufficient airtime usage to merit additional frequencies.  A showing of airtime usage, excluding telephone interconnect air time, during the peak busy hour greater than 70 percent per channel on three consecutive days will be required to satisfy loading criteria.

4.4.3  Slow Growth

All systems in the 821-824/866-869 MHz bands under this plan will be slow growth in accordance with Section 90-629 of the Commission rules.

4.5  Use of Long Range Communications

During incidents of major proportions, where Public Safety requirements might include the need for long-range communications in and out of a disaster area, alternate radio communications plans are to be addressed by Primary Public Safety agencies within this sub-region.  These agencies should integrate the appropriate interface to the long distance communications providers.  Such long distance radio communications might be amateur radio operations, satellite communications and/or long range emergency preparedness communications systems, any of or all of which should be incorporated as part of the communications plans of those lead agencies.  They then could provide the means to communicate outside the area for themselves and the smaller agencies who might need assistance.  Instances as addressed in the National Public Safety Planning Advisory Committee’s Plan, such as earthquakes, hurricanes, floods, widespread forest fires, or nuclear reactor problems could be a cause for such long-range communications needs.

4.6  Expansion of Existing Systems

Existing systems that are to be expanded to include the frequency bands of 821-824/866-869 MHz will have the mobile radios “grandfathered”, provided that they are modified in conformance with the Memorandum Opinion and Order, FCC Docket 87-112.  Primarily this involves reducing the modulation to +/- 4 KHz. Existing base stations in the frequency bands 806-821/851-866 MHz may not be used in the frequency bands of 821-824/866-869 MHz.

5.0  IMPLEMENTATION AND PROCEDURES

5.1  Notification

Several methods of notification were used to invite interested parties to participate in the development of this plan.  Initially, personal contact was made by the “convener” to all of the major State agency communications users in the State of Missouri.  Announcements were made at various group meetings.

Supplemental to the personal contact, all APCO Chapter members and a large number of other interested parties who had requested notification were sent letters of invitation.

During the initial meeting, names, addresses, and telephone numbers of those individuals present were taken. (Appendix A)

When the work on the plan was completed, a final planning committee meeting was called.  This meeting was held at the Missouri State Highway Patrol Headquarters on November 12, 1992.  Each member of the planning committee was presented with a draft copy of the plan for study.  A copy of the draft, with changes made at the meeting of November 12, 1992, was mailed to each member of the committee including those not present at the meeting.  Review of the draft prompted some additional replies and further changes to the plan.  Those changes were mailed to all members of the committee, together with a ballot for them to execute a vote of approval or rejection.  By virtue of a decision made at the meeting of November 12, 1992, a simple majority was considered to be sufficient to approve the plan.

5.2  Frequency Allocation Process

The method used for “packing” Region 24 was the APCO computerized method.  The approximate geographical location for the center of each county, in latitude and longitude were provided along with the environmental type of the county and the approximate radius to cover the county lines.

This allocation is the minimum and only applies to counties with a population of 20,000 or less.  One additional channel is allocated for each additional 20,000 of population.

5.3  Frequency Allocation Map

Attached (Appendix K) is the date, or packing plan generated by APCO via the computerized packing program.  The plan took adjacent regions into consideration, in addition, letters of concurrence were sent.

5.5  Expansion of Initial Allocation

In the event that the allocation for any county becomes depleted, the Region Review Committee shall meet to make further allocations to said county.  Should this occur, the applying agency or entity shall submit the proper license and coordination applications with all applicable fees, as in any other licensing request.  Allocations will be made based on the initial frequency allocation plan as mentioned above, taking into consideration the channels which were returned to the reserve pool.

5.6  Prioritization of Applicants

A very simple method of prioritization has been chosen for use in this Region.  As there is no unmet spectrum requirement, there appears to be no great need for prioritization.  In order to facilitate future problems which may arise, the following rating system shall be used.

Prioritization shall be done according to a final score, based on applicant criteria.  The highest score, in points, shall be given priority in a situation where spectrum is insufficient to fulfill the needs of all.

Public Safety Agencies........................................2 points

Public Services Agencies.....................................1 point

Multi-agency Systems..........................................2 points

Multi-agency/Multi-Jurisdictional Systems         3 points

Single Agency/Jurisdiction Systems                    1 point

5.7  Appeal Process

At any time, any applicant may appeal an allocation, rejection, or any limits placed on a particular application for any reason.  The appeal process has two levels: the Region Review Committee and the FCC.  An applicant who decides to appeal a rejection should initiate that appeal immediately upon notification of rejection.  In the event that an appeal reaches the FCC, their decision will be final and binding upon all parties.

6.0  THE REGION 24 PLANNING COMMITTEE

CHAIRPERSON:

John R. Gerke

City of St. Louis

Communications Division

4971 Oakland Ave,

St. Louis, Missouri 63110

Phone:  314-533-5802

Fax:      314-534-7059

THE REGIONAL PLANNING COMMITTEE

ALL THE MEMBERS NAMES, ADDRESSES, AGENCIES AFFILIATION, PHONE NUMBERS

(See Appendix H)

APPENDIX A

Abart, Frank	Boone County Public Works	314-449-8515

Ahrens, Ken	MSHP, Communications	314-751-3747

Arthur, Vance	Motorola C & E	913-491-1090

Badgett, Chuck	St. Louis City Fire Dept.	314-533-4175

Bangs, Neil	Belton Fire Dept.	816-331-7969

Bauer, William 	North Central County Fire Alarm	314-428-1134

Bland, David	G.E., Independence, Mo.	913-642-9700

Bolden, David	Jefferson City Ambulance Service	314-635-7141

Bond, Kevin	Pettis County Sheriff’s Dept.	816-827-0052

Britts, Kimberly	Cole County Sheriff’s Dept.	314-634-9160

Byrne, Tom	Clayton Police Dept	314-727-4130

Cain, Bill	Butler Police Dept.	816-679-6131

Clark, Lee	FEMA Region VII, K.C., Mo.	816-283-7074

Davisson, David	Benton County EMS	816-438-6779

Deck, David	Breech Paramedics	417-532-2136

Deitch, Bobby	Johnson County Central Dispatch	816-747-2265

Dennis, Clay	Benton County Sheriff’s Dept.	816-438-5252

Dometrorch, Shelly	Boone County Sheriff’s Dept.	314-874-6226

Eador, John	Nevada, Mo. Police Dept.	417-667-7848

Edwards, David	Camden County Sheriff’s Dept.	314-346-2243

Edwards, David	MEMSA (Mo. EMA Assoc.)	314-449-5954

Gerke, John	St. Louis PD		34-533-5802

Gray, Mike	Mo. Dept. of Health/EMS	314-751-6356

Grideley, Timothy	MAST Ambulance, K.C., MO.	8168-361-2600

Gulick, Barbara	Kansas City, Mo. PD	816-234-5110

Harris, Frank	Creve Coeur PD	314-569-5024

Hart, Bill	Motorola C&E	913-491-1090

Heigl, Val	Overland PD		314-428-1221

Henry, Ron	Clinton County Sheriff’s Dept	816-539-3777

Hensley, Tom	Mo. Ambulance Dir. Network	816-539-2317

Hill, Charles	Kansas City, Mo. PD	816-231-1997

Hood, Ron	Mo. Conversation Dept.	314-751-4115

Johnson, Kieth	Henry Co. Sheriff’s Dept.	816-885-5587

Johnston, Phil	Springfield Fire Dept.	417-864-1041

Jones, J.L.	Greene Co. Sheriff’s Dept.	417-868-4040

Katz, Steve	Kustom Electronics	913-383-1337

Kayser, Don	Creve Coeur PD	314-432-8000

Kearns, Tom	G.E., Overland Park, Ks.	916-642-9700

Keene, C. Richard	City of Kansas City, Mo.	816-274-1405

Knittig, Bill	Central County Emergency Dispatch	314-394-5443

Krider, Leroy	Kaysinger Basin RPC, Clinton	8186-885-3393

Lachner, Don	Mo. State Water Patrol	314-751-2063

Lecuru, Rhonda	Jefferson City, PD	314-634-6400

Lewis, John	MSHP, Communications	314-751-3747

Ludwig, Gary	EMS, St. Louis	314-645-9160

Luna, Verlin	MSHP, Communications	314-751-3747

Lundsted, Jim	MSHP, Communications	314-751-3747

McGraw, Dan	City of Independence, Mo.	816-836-8300

McNaron, Jim	Benton County Sheriff’s Dept.	816-438-5252

Miles, Vic	Kansas City, Mo. Fire Dept.	816-472-1823

Miller, Eldon	Belton Emergency Management	816-331-7969

Montgomery, Jerry	University City PD	314-725-2211

Morris, Art	Nevada, Mo. Police Dept.	417-667-7848

Oliver, Mona	Columbia/Boone County Joint Comm.	314-874-7401

Osterhoudt, Darrell	Mo. Dept. of Natural Resources	314-751-1187

Ostrander, John	Mo. Highway & Transp. Dept.	314-751-4675

O’Connor, John	Motorola C&E	913-491-1090

Peters, Kenny	Mo. Dept of Health/EMS	314-751-6356

Redman, Michael G.	St. Louis County PD	314-889-2362

Riley, W.O.	Raytown PD		816-757-6016

Robertson, Ed	Osage Council on Criminal Justice	816-679-5650

Sapp, Steven	Columbia/Boone County Joint Comm.	314-874-7471

Schurman, Dick	Joplin PD		417-623-3131

Scott, Sharon	FCC, K.C., Mo.	816-926-5111

Scrivner, Kent	JCPD & Mo. Police Chiefs Assoc.	314-634-6400

Siemers, Michael	St. Louis PD		314-444-5502

Smith Sherie	Maryland Heights PD	314-298-8700

Spencer, Paul DVM	Mo. Vet. Med. Assoc.	314-751-2334

Stout, Elisabeth	Natl. Park Service, St. Louis, Mo.	314-425-5095

Tate, Roy	Greene County Sheriff’s Dept.	417-868-4040

Teeter, Ken	Mo. DNR, EER, Jefferson City	314-751-7929

Townley, Audry	EMS, St. Louis, Mo.	314-622-5160

Turkisher, Dan	Motorola C&E	913-491-1090

Vreeland, George D.	G.E. Mobile		314-432-1336

Weilbacher, Mike	REJIS, St. Louis, Mo.	314-535-1950

West, Bob	Camden County Sheriff’s Dept.	314-346-2243

Whitaker, Paul	K.C., Mo. Area Hospital Assoc.	816-941-3800

Wilcox, Samuel	University City PD	314-725-2211

Zelch, Vernon	Sullivan PD		314-468-8001

Zerr, Rod	St. Charles County Ambulance Dist.	314-947-4187

Zimmer, Mark	Rock Hill PD	314-962-6600

APPENDIX B

Aldis, Paul	unknown

Albera, Peter J.	Motorola C&E

Bauer, William	No. Central County Fire Alarm	314-428-1134

Bond, Kevin C.	Pettis County Sheriff’s Dept.	816-827-0052

Brenneke, Sam	unknown

Clark, Lee	FEMA, Region VII, K.C., Mo.	816-283-7074

Davisson, David	Benton County EMS	816-438-6779

Edwards, David	MEMSA (Mo. EMA Assoc.)	314-449-5954

Gerke, John	St. Louis PD		314-533-5802

Gray, Mike	Missouri Bureau of EMS	314-751-6356

Gridley, Timothy S.	MAST Ambulance, K.C., Mo.	816-361-2600

Henry, Ron	Clinton County Sheriff’s Dept.	816-539-3777

Hensley, Tom	Mo. Ambulance Dir. Network	816-539-2317

Hook, Ron	Mo. Conservation Dept.	314-751-4115

Johnston, Phil	Springfield Fire Dept.	417-864-1041

Keene, Dick	City of Kansas City, Mo.	8186-274-1405

Knittig, Bill	Central County Emerg. Dispatch	314-394-5443

Krider, Leroy	Kaysinger Basin RPC, Clinton	816-885-3393

Lachner, Don	Mo. State Water Patrol	314-751-2063

Lundsted, Jim	MSHP Communications	314-751-3747

Morris, Art	Nevada, Mo. PD	417-667-7848

Oliver, Mona	Columbia/Boone County Joint Comm.	314-874-7401

Ostrander, John	Mo. Highway & Transp. Dept.	314-751-4675

Peters, Kenny	Missouri Bureau of EMS	314-751-6356

Ryan, Tom	unknown

Sapp, Steven	Columbia/Boone County Joint Comm.	314-874-7471

Schurman, Dick	Joplin PD		417-623-3131

Scrivner, Kent	Jefferson City, Mo. PD & Police Chief’s Assoc.	314-634-6400

Siemers, Lt. Michael	St. Louis, PD		314-444-5502

Smith, Sherie	Maryland Heights PD	314-298-8700

Teeter, Ken	Mo. DNR, EER, Jefferson City	314-751-7929

Thayer, Michael	unknown

Zerr, Rod	St. Charles County Ambulance Dist.	314-947-6600

APPENDIX C

Bauer, William	No. Central County Fire Alarm	314-428-1134

Bond, Kevin C.	Pettis County Sheriff’s Dept.	816-827-0052

Davisson, David	Benton County EMS	816-438-6779

Gerke, John	St. Louis PD		314-533-5802

Gray, Mike	Mo. Bureau of EMS	314-751-6356

Gridley, Timothy S.	MAST Ambulance, K.C., Mo.	816-361-2600

Hook, Ron	Mo. Conservation Dept.	314-751-4115

Johnston, Phil	Springfield Fire Dept.	417-864-1041

Knittig, Bill	Central County Emergency Disp.	314-394-5443

Krider, Leroy	Kaysinger Basin RPC, Clinton	816-885-3393

Lundsted, Jim 	MSHP Communications	314-751-3747

Morris, Art	Nevada PD		417-667-7848

Oliver, Mona	Columbia/Boone County Joint Comm.	314-874-7401

Ostrander, John	Mo. Highway & Transportation Dept.	314-751-4675

Peters, Kenny	Missouri Bureau of EMS	314-751-6356

Sapp, Steven	Columbia/Boone County Joint Comm.	314-874-7471

Scrivner, Kent	Jefferson City PD & Police Chiefs Assoc.	314-634-6400

Smith, Sherie	Maryland Heights PD	314-298-8700

APPENDIX D

Albera, Peter J.	Motorola C&E

Bauer, William	North Central County Fire Alarm	314-428-1134

Bond, Kevin C.	Pettis County Sheriff’s Dept.	816-827-0052

Brenneke, Sam	unknown

Davisson, David	Benton County EMS	816-438-6779

Gerke, John	St. Louis PD		314-533-5802

Gray, Mike	Missouri Bureau of EMS	314-751-6356

Gridley, Timothy S	MAST Ambulance, K.C., Mo.	816-361-2600

Hook, Ron	Mo. Conservation Dept.	314-751-4115

Johnston, Phil	Springfield Fire Dept.	417-864-1041

Knittig, Bill	Central County Emerge Disp.	314-394-5443

Krider, Leroy	Kaysinger Basin RPC, Clinton	816-885-3393

Lundsted, Jim	MSHP Communications	314-751-3747

Morris, Art	Neveda PD		417-667-7848

Oliver, Mona	Columbia/Boone County Joint Comm.	314-874-7401

Ostrander, John	Mo. Highway & Transp. Dept.	314-751-4675

Peters, Kenny	Missouri Bureau of EMS	314-751-6356

Ryan, Tom	unknown

Sapp, Steven	Columbia/Boone County Joint Comm.	314-874-7471

Scrivner, Kent	Jefferson City PD & Police Chiefs Assoc.	314-634-6400

Smith Sherie	Maryland Heights PD	314-298-8700

Thayer, Mike	unknown

APPENDIX E

Bauer, Mike	Columbia/Boone County Joint Comm.	314-874-7401

Bauer, William	North Central County Fire Alarm	314-428-1134

Brooks, John	Dir. Springfield Mo. Comm.	417-874-1718

Davisson, David	Benton County EMS	816-438-6779

Dollus, Tom	M&T Div. Mo. Highway & Transp. Dept.	314-751-2845

Estabrooks, Laura	Columbia/Boone County Joint Comm.	314-874-7401

Gerke, John	St. Louis City	314-533-5802

Gray, Mike 	Missouri Bureau of EMS	314-751-6356

Hill, Charles	Kansas City PD	816-231-1997

Huenik, Hans	Mo. State Water Patrol	314-751-3333

Knittig, Bill	Central County Emerg. Disp.	314-394-5443

Knobbe, Steve 	M&T Div. Mo. Highway & Transp Dept.	314-751-2822

Lachner, Don	Mo. State Water Patrol	314-751-2063

Lundsted, Jim	MSHP Communications	314-751-3747

Miles, Vic	Kansas City Fire Dept.	816-471-1823

Ostrander, john	Mo. Highway & Transp. Dept	314-751-4675

Redman, Michael	St. Louis County PD	314-889-2362

APPENDIX F

Dabell, Dr. David	Med. Dir. St. Johns Hospital	314-569-6000

Dietrich, Robert	Gateway Ambulance	314-351-4702

Dougherty, Terry	Abbott Ambulance	314-963-1000

Dreifuss, Rodney	St. Louis City EMS	314-645-9160

Stump, Richard	Sema Mo.		314-751-9583

APPENDIX G

Bauer, Mike	Columbia/Boone County Joint Comm	314-874-7401

Barrow, John	Gateway Ambulance	314-351-4702

Bauer, William	North Central County Fire Alarm	314-428-1134

Biggerstaff, Jim	MSHP Communications	314-751-3747

Brooks, John	Dir. Springfield Mo. Comm	417-864-1718

Dabell, Dr. David	Med. Dir. St. Johns Hosp.	314-569-6000

Davisson, David	Benton County EMS	816-438-6779

Dollus, Tom	M&T Div. Mo. Highway & Transp. Dept.	314-751-2845

Gerke, John	St. Louis City	314-533-5802

Gray, Mike	Missouri Bureau of EMS	314-751-6356

Hill, Charles	Kansas City PD	816-231-1997

Huenik, Hans	Mo. State Water Patrol	314-751-3333

Huffman, John	Abbott Ambulance	314-963-1000

Knobbe, Steve	M&T Div. Mo. Highway & Transp. Dept.	314-751-2822

Lachner, Don	Mo. State Water Patrol	314-751-2063

Ludwig, Gary	St. Louis City EMS	314-645-9160

Lundsted, Jim	MSHP Communications	314-751-3747

Miles, Vic	Kansas City Fire Dept.	816-471-1823

Ostrander, John	Mo. Highway & Transp. Dept.	314-751-4675

Peters, Ken	Missouri Bureau of EMS	314-751-6356

Redman, Michael	St. Louis County PD	314-889-2362

Smith, Sherie	Maryland Heights PD	314-298-8700

Stump, Richard	SEMA, Mo.		314-751-9583

APPENDIX H

Barrow, John

Gateway Ambulance

6492 Gravois

St. Louis, Mo. 63116

314-351-4702

Bauer, Bill

North Central Fire Alarm

3280 Mcgibbon Dr.

St. Louis, Mo. 63114

314-428-1134

Biggerstaff, Jim

MSHP

P O Box 568

Jefferson City, Mo. 65102

314-751-3747

Brooks, John

Springfield, Mo. Emerg. Comm.

319 E. Chesnut Expressway

Springfield, Mo. 65802

417-864-1718

Crouch, Russell

St. Louis EMS

2634 Hampton Ave.

St. Louis, Mo. 63108

314-531-1000

Gerke, John

City of St. Louis

Communications Division

4971 Oakland

St. Louis, Mo. 63110

Gray, Mike

Missouri Bureau of EMS

P O Box 570

Jefferson City, Mo 65102

314-751-6356

Hill, Charles

Kansas City, Mo. PD

3324 E. 12th St.

Kansas City, Mo. 64127

816-231-1997

APPENDIX I

Huenik, Hans

Mo. State Water Patrol

P O Box 1368

Jefferson City, Mo 65102-1368

314-751-3333

Huffman, John

Abbott Ambulance

8401 Henley Ind. Ct.

St. Louis, Mo.  63144

314-963-1000

Lachner, Don

Mo. State Water Patrol

P O Box 1368

Jefferson City, Mo. 65102

314-751-2063

Lundsted, Jim

MSHP

P O Box 568

Jefferson City, Mo. 65102

314-751-3747

Miles, Vic

Kansas City, Mo. Fire Dept

414 E. 22nd St.

Kansas city, Mo. 64108-2602

816-471-1823

Peters, Ken

Missouri Bureau of EMS

P O Box 570

Jefferson City, Mo. 65102

314-751-6356

Redman, Michael

St. Louis County PD Communications

7900 Forsythe Blvd.

St. Louis, Mo. 63105

341-889-2362

Scott Brian

Mo. Highway & Transp. Dept.

P O Box 270

Jefferson City, MO. 65102

314-751-2822

Townley, A.J.

St. Louis City EMS

2634 Hampton Ave.

St. Louis, Mo. 63139

314-645-9160

APPENDIX J

Included in this appendix are the responses from the Regions Adjacent to Region 24, State of Missouri.

Considering that some Regions have given conditional approval resulting from some very sensitive areas

of frequency packing, it is my intention and the intention of the Committee to engage in very close pre-

allocation coordination with all adjacent Regions.  (Docket 93-131)

Included also is a generic copy of the cover letter sent with the draft of our plan.  All copies were sent

certified and I have Received all return receipts.

(SEE PDF (SCANNED) COPIES OF THE FOLLOWING CORRESPONDENCE)

January 2, 1993    from the Chairman of Region 24 John Gerke to surrounding states: Final Draft

of the Region 24 800 MHz plan for the State of Missouri.

January 14, 1993  from Tennessee Region 24 advising of concurrence but bringing attention to

two potential frequency conflicts.

January 15, 1993  from Illinois FCC Region 13 advising of some frequency conflicts.

January 15, 1993  from Nebraska Region 26 advising of no conflicts with the plan.

January 24, 1993  from OK HP advising of no comments or conflicts with the plan.

undated                 from Region 16 advising of some frequency conflicts.

Appendix K – Anticipated 800 MHz NPSPAC Band Plan definitions, correllations and channel interpretations between previous and new channel allocations.

Correlation between NPSPAC old channel plan and new channel plan with corresponding channel numbers, guard channels, etc.

| ORIG FCC CH NUMBER | ORIG FREQ |  | NEW FCC CH NUMBER | NEW FREQ |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 601 | 866.0125 | ICALL | 1 | 851.0125 |  |  |
|  |  |  | guard | 851.0250 |  |  |
| 602 | 866.0375 |  | 2 | 851.0375 |  |  |
| 603 | 866.0500 |  | 3 | 851.0500 |  |  |
| 604 | 866.0625 |  | 4 | 851.0625 |  |  |
| 605 | 866.0750 |  | 5 | 851.0750 |  |  |
| 606 | 866.0875 |  | 6 | 851.0875 |  |  |
| 607 | 866.1000 |  | 7 | 851.1000 |  |  |
| 608 | 866.1125 |  | 8 | 851.1125 |  |  |
| 609 | 866.1250 |  | 9 | 851.1250 |  |  |
| 610 | 866.1375 |  | 10 | 851.1375 |  |  |
| 611 | 866.1500 |  | 11 | 851.1500 |  |  |
| 612 | 866.1625 |  | 12 | 851.1625 |  |  |
| 613 | 866.1750 |  | 13 | 851.1750 |  |  |
| 614 | 866.1875 |  | 14 | 851.1875 |  |  |
| 615 | 866.2000 |  | 15 | 851.2000 |  |  |
| 616 | 866.2125 |  | 16 | 851.2125 |  |  |
| 617 | 866.2250 |  | 17 | 851.2250 |  |  |
| 618 | 866.2375 |  | 18 | 851.2375 |  |  |
| 619 | 866.2500 |  | 19 | 851.2500 |  |  |
| 620 | 866.2625 |  | 20 | 851.2625 |  |  |
| 621 | 866.2750 |  | 21 | 851.2750 |  |  |
| 622 | 866.2875 |  | 22 | 851.2875 |  |  |
| 623 | 866.3000 |  | 23 | 851.3000 |  |  |
| 624 | 866.3125 |  | 24 | 851.3125 |  |  |
| 625 | 866.3250 |  | 25 | 851.3250 |  |  |
| 626 | 866.3375 |  | 26 | 851.3375 |  |  |
| 627 | 866.3500 |  | 27 | 851.3500 |  |  |
| 628 | 866.3625 |  | 28 | 851.3625 |  |  |
| 629 | 866.3750 |  | 29 | 851.3750 |  |  |
| 630 | 866.3875 |  | 30 | 851.3875 |  |  |
| 631 | 866.4000 |  | 31 | 851.4000 |  |  |
| 632 | 866.4125 |  | 32 | 851.4125 |  |  |
| 633 | 866.4250 |  | 33 | 851.4250 |  |  |
| 634 | 866.4375 |  | 34 | 851.4375 |  |  |
| 635 | 866.4500 |  | 35 | 851.4500 |  |  |
| 636 | 866.4625 |  | 36 | 851.4625 |  |  |
| 637 | 866.4750 |  | 37 | 851.4750 |  |  |
| 638 | 866.4875 |  | 38 | 851.4875 |  |  |
|  |  |  | guard | 851.5000 |  |  |
| 639 | 866.5125 | ITAC1 | 39 | 851.5125 |  |  |
|  |  |  | guard | 851.5250 |  |  |
| 640 | 866.5375 |  | 40 | 851.5375 |  |  |
| 641 | 866.5500 |  | 41 | 851.5500 |  |  |
| 642 | 866.5625 |  | 42 | 851.5625 |  |  |
| 643 | 866.5750 |  | 43 | 851.5750 |  |  |
| 644 | 866.5875 |  | 44 | 851.5875 |  |  |
| 645 | 866.6000 |  | 45 | 851.6000 |  |  |
| 646 | 866.6125 |  | 46 | 851.6125 |  |  |
| 647 | 866.6250 |  | 47 | 851.6250 |  |  |
| 648 | 866.6375 |  | 48 | 851.6375 |  |  |
| 649 | 866.6500 |  | 49 | 851.6500 |  |  |
| 650 | 866.6625 |  | 50 | 851.6625 |  |  |
| 651 | 866.6750 |  | 51 | 851.6750 |  |  |
| 652 | 866.6875 |  | 52 | 851.6875 |  |  |
| 653 | 866.7000 |  | 53 | 851.7000 |  |  |
| 654 | 866.7125 |  | 54 | 851.7125 |  |  |
| 655 | 866.7250 |  | 55 | 851.7250 |  |  |
| 656 | 866.7375 |  | 56 | 851.7375 |  |  |
| 657 | 866.7500 |  | 57 | 851.7500 |  |  |
| 658 | 866.7625 |  | 58 | 851.7625 |  |  |
| 659 | 866.7750 |  | 59 | 851.7750 |  |  |
| 660 | 866.7875 |  | 60 | 851.7875 |  |  |
| 661 | 866.8000 |  | 61 | 851.8000 |  |  |
| 662 | 866.8125 |  | 62 | 851.8125 |  |  |
| 663 | 866.8250 |  | 63 | 851.8250 |  |  |
| 664 | 866.8375 |  | 64 | 851.8375 |  |  |
| 665 | 866.8500 |  | 65 | 851.8500 |  |  |
| 666 | 866.8625 |  | 66 | 851.8625 |  |  |
| 667 | 866.8750 |  | 67 | 851.8750 |  |  |
| 668 | 866.8875 |  | 68 | 851.8875 |  |  |
| 669 | 866.9000 |  | 69 | 851.9000 |  |  |
| 670 | 866.9125 |  | 70 | 851.9125 |  |  |
| 671 | 866.9250 |  | 71 | 851.9250 |  |  |
| 672 | 866.9375 |  | 72 | 851.9375 |  |  |
| 673 | 866.9500 |  | 73 | 851.9500 |  |  |
| 674 | 866.9625 |  | 74 | 851.9625 |  |  |
| 675 | 866.9750 |  | 75 | 851.9750 |  |  |
| 676 | 866.9875 |  | 76 | 851.9875 |  |  |
|  |  |  | guard | 852.0000 |  |  |
| 677 | 867.0125 | ITAC2 | 77 | 852.0125 |  |  |
|  |  |  | guard | 852.0250 |  |  |
| 678 | 867.0375 |  | 78 | 852.0375 |  |  |
| 679 | 867.0500 |  | 79 | 852.0500 |  |  |
| 680 | 867.0625 |  | 80 | 852.0625 |  |  |
| 681 | 867.0750 |  | 81 | 852.0750 |  |  |
| 682 | 867.0875 |  | 82 | 852.0875 |  |  |
| 683 | 867.1000 |  | 83 | 852.1000 |  |  |
| 684 | 867.1125 |  | 84 | 852.1125 |  |  |
| 685 | 867.1250 |  | 85 | 852.1250 |  |  |
| 686 | 867.1375 |  | 86 | 852.1375 |  |  |
| 687 | 867.1500 |  | 87 | 852.1500 |  |  |
| 688 | 867.1625 |  | 88 | 852.1625 |  |  |
| 689 | 867.1750 |  | 89 | 852.1750 |  |  |
| 690 | 867.1875 |  | 90 | 852.1875 |  |  |
| 691 | 867.2000 |  | 91 | 852.2000 |  |  |
| 692 | 867.2125 |  | 92 | 852.2125 |  |  |
| 693 | 867.2250 |  | 93 | 852.2250 |  |  |
| 694 | 867.2375 |  | 94 | 852.2375 |  |  |
| 695 | 867.2500 |  | 95 | 852.2500 |  |  |
| 696 | 867.2625 |  | 96 | 852.2625 |  |  |
| 697 | 867.2750 |  | 97 | 852.2750 |  |  |
| 698 | 867.2875 |  | 98 | 852.2875 |  |  |
| 699 | 867.3000 |  | 99 | 852.3000 |  |  |
| 700 | 867.3125 |  | 100 | 852.3125 |  |  |
| 701 | 867.3250 |  | 101 | 852.3250 |  |  |
| 702 | 867.3375 |  | 102 | 852.3375 |  |  |
| 703 | 867.3500 |  | 103 | 852.3500 |  |  |
| 704 | 867.3625 |  | 104 | 852.3625 |  |  |
| 705 | 867.3750 |  | 105 | 852.3750 |  |  |
| 706 | 867.3875 |  | 106 | 852.3875 |  |  |
| 707 | 867.4000 |  | 107 | 852.4000 |  |  |
| 708 | 867.4125 |  | 108 | 852.4125 |  |  |
| 709 | 867.4250 |  | 109 | 852.4250 |  |  |
| 710 | 867.4375 |  | 110 | 852.4375 |  |  |
| 711 | 867.4500 |  | 111 | 852.4500 |  |  |
| 712 | 867.4625 |  | 112 | 852.4625 |  |  |
| 713 | 867.4750 |  | 113 | 852.4750 |  |  |
| 714 | 867.4875 |  | 114 | 852.4875 |  |  |
|  |  |  | guard | 852.5000 |  |  |
| 715 | 867.5125 | ITAC3 | 115 | 852.5125 |  |  |
|  |  |  | guard | 852.5250 |  |  |
| 716 | 867.5375 |  | 116 | 852.5375 |  |  |
| 717 | 867.5500 |  | 117 | 852.5500 |  |  |
| 718 | 867.5625 |  | 118 | 852.5625 |  |  |
| 719 | 867.5750 |  | 119 | 852.5750 |  |  |
| 720 | 867.5875 |  | 120 | 852.5875 |  |  |
| 721 | 867.6000 |  | 121 | 852.6000 |  |  |
| 722 | 867.6125 |  | 122 | 852.6125 |  |  |
| 723 | 867.6250 |  | 123 | 852.6250 |  |  |
| 724 | 867.6375 |  | 124 | 852.6375 |  |  |
| 725 | 867.6500 |  | 125 | 852.6500 |  |  |
| 726 | 867.6625 |  | 126 | 852.6625 |  |  |
| 727 | 867.6750 |  | 127 | 852.6750 |  |  |
| 728 | 867.6875 |  | 128 | 852.6875 |  |  |
| 729 | 867.7000 |  | 129 | 852.7000 |  |  |
| 730 | 867.7125 |  | 130 | 852.7125 |  |  |
| 731 | 867.7250 |  | 131 | 852.7250 |  |  |
| 732 | 867.7375 |  | 132 | 852.7375 |  |  |
| 733 | 867.7500 |  | 133 | 852.7500 |  |  |
| 734 | 867.7625 |  | 134 | 852.7625 |  |  |
| 735 | 867.7750 |  | 135 | 852.7750 |  |  |
| 736 | 867.7875 |  | 136 | 852.7875 |  |  |
| 737 | 867.8000 |  | 137 | 852.8000 |  |  |
| 738 | 867.8125 |  | 138 | 852.8125 |  |  |
| 739 | 867.8250 |  | 139 | 852.8250 |  |  |
| 740 | 867.8375 |  | 140 | 852.8375 |  |  |
| 741 | 867.8500 |  | 141 | 852.8500 |  |  |
| 742 | 867.8625 |  | 142 | 852.8625 |  |  |
| 743 | 867.8750 |  | 143 | 852.8750 |  |  |
| 744 | 867.8875 |  | 144 | 852.8875 |  |  |
| 745 | 867.9000 |  | 145 | 852.9000 |  |  |
| 746 | 867.9125 |  | 146 | 852.9125 |  |  |
| 747 | 867.9250 |  | 147 | 852.9250 |  |  |
| 748 | 867.9375 |  | 148 | 852.9375 |  |  |
| 749 | 867.9500 |  | 149 | 852.9500 |  |  |
| 750 | 867.9625 |  | 150 | 852.9625 |  |  |
| 751 | 867.9750 |  | 151 | 852.9750 |  |  |
| 752 | 867.9875 |  | 152 | 852.9875 |  |  |
|  |  |  | guard | 853.0000 |  |  |
| 753 | 868.0125 | ITAC4 | 153 | 853.0125 |  |  |
|  |  |  | guard | 853.0250 |  |  |
| 754 | 868.0375 |  | 154 | 853.0375 |  |  |
| 755 | 868.0500 |  | 155 | 853.0500 |  |  |
| 756 | 868.0625 |  | 156 | 853.0625 |  |  |
| 757 | 868.0750 |  | 157 | 853.0750 |  |  |
| 758 | 868.0875 |  | 158 | 853.0875 |  |  |
| 759 | 868.1000 |  | 159 | 853.1000 |  |  |
| 760 | 868.1125 |  | 160 | 853.1125 |  |  |
| 761 | 868.1250 |  | 161 | 853.1250 |  |  |
| 762 | 868.1375 |  | 162 | 853.1375 |  |  |
| 763 | 868.1500 |  | 163 | 853.1500 |  |  |
| 764 | 868.1625 |  | 164 | 853.1625 |  |  |
| 765 | 868.1750 |  | 165 | 853.1750 |  |  |
| 766 | 868.1875 |  | 166 | 853.1875 |  |  |
| 767 | 868.2000 |  | 167 | 853.2000 |  |  |
| 768 | 868.2125 |  | 168 | 853.2125 |  |  |
| 769 | 868.2250 |  | 169 | 853.2250 |  |  |
| 770 | 868.2375 |  | 170 | 853.2375 |  |  |
| 771 | 868.2500 |  | 171 | 853.2500 |  |  |
| 772 | 868.2625 |  | 172 | 853.2625 |  |  |
| 773 | 868.2750 |  | 173 | 853.2750 |  |  |
| 774 | 868.2875 |  | 174 | 853.2875 |  |  |
| 775 | 868.3000 |  | 175 | 853.3000 |  |  |
| 776 | 868.3125 |  | 176 | 853.3125 |  |  |
| 777 | 868.3250 |  | 177 | 853.3250 |  |  |
| 778 | 868.3375 |  | 178 | 853.3375 |  |  |
| 779 | 868.3500 |  | 179 | 853.3500 |  |  |
| 780 | 868.3625 |  | 180 | 853.3625 |  |  |
| 781 | 868.3750 |  | 181 | 853.3750 |  |  |
| 782 | 868.3875 |  | 182 | 853.3875 |  |  |
| 783 | 868.4000 |  | 183 | 853.4000 |  |  |
| 784 | 868.4125 |  | 184 | 853.4125 |  |  |
| 785 | 868.4250 |  | 185 | 853.4250 |  |  |
| 786 | 868.4375 |  | 186 | 853.4375 |  |  |
| 787 | 868.4500 |  | 187 | 853.4500 |  |  |
| 788 | 868.4625 |  | 188 | 853.4625 |  |  |
| 789 | 868.4750 |  | 189 | 853.4750 |  |  |
| 790 | 868.4875 |  | 190 | 853.4875 |  |  |
| 791 | 868.5000 |  | 191 | 853.5000 |  |  |
| 792 | 868.5125 |  | 192 | 853.5125 |  |  |
| 793 | 868.5250 |  | 193 | 853.5250 |  |  |
| 794 | 868.5375 |  | 194 | 853.5375 |  |  |
| 795 | 868.5500 |  | 195 | 853.5500 |  |  |
| 796 | 868.5625 |  | 196 | 853.5625 |  |  |
| 797 | 868.5750 |  | 197 | 853.5750 |  |  |
| 798 | 868.5875 |  | 198 | 853.5875 |  |  |
| 799 | 868.6000 |  | 199 | 853.6000 |  |  |
| 800 | 868.6125 |  | 200 | 853.6125 |  |  |
| 801 | 868.6250 |  | 201 | 853.6250 |  |  |
| 802 | 868.6375 |  | 202 | 853.6375 |  |  |
| 803 | 868.6500 |  | 203 | 853.6500 |  |  |
| 804 | 868.6625 |  | 204 | 853.6625 |  |  |
| 805 | 868.6750 |  | 205 | 853.6750 |  |  |
| 806 | 868.6875 |  | 206 | 853.6875 |  |  |
| 807 | 868.7000 |  | 207 | 853.7000 |  |  |
| 808 | 868.7125 |  | 208 | 853.7125 |  |  |
| 809 | 868.7250 |  | 209 | 853.7250 |  |  |
| 810 | 868.7375 |  | 210 | 853.7375 |  |  |
| 811 | 868.7500 |  | 211 | 853.7500 |  |  |
| 812 | 868.7625 |  | 212 | 853.7625 |  |  |
| 813 | 868.7750 |  | 213 | 853.7750 |  |  |
| 814 | 868.7875 |  | 214 | 853.7875 |  |  |
| 815 | 868.8000 |  | 215 | 853.8000 |  |  |
| 816 | 868.8125 |  | 216 | 853.8125 |  |  |
| 817 | 868.8250 |  | 217 | 853.8250 |  |  |
| 818 | 868.8375 |  | 218 | 853.8375 |  |  |
| 819 | 868.8500 |  | 219 | 853.8500 |  |  |
| 820 | 868.8625 |  | 220 | 853.8625 |  |  |
| 821 | 868.8750 |  | 221 | 853.8750 |  |  |
| 822 | 868.8875 |  | 222 | 853.8875 |  |  |
| 823 | 868.9000 |  | 223 | 853.9000 |  |  |
| 824 | 868.9125 |  | 224 | 853.9125 |  |  |
| 825 | 868.9250 |  | 225 | 853.9250 |  |  |
| 826 | 868.9375 |  | 226 | 853.9375 |  |  |
| 827 | 868.9500 |  | 227 | 853.9500 |  |  |
| 828 | 868.9625 |  | 228 | 853.9625 |  |  |
| 829 | 868.9750 |  | 229 | 853.9750 |  |  |
| 830 | 868.9875 |  | 230 | 853.9875 |  |  |
|  |  |  | guard | 854.0000 |  |  |

Appendix L Revised Channel Allotments

| required Can be coordinated with Region 13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel allotments in Yellow are alterations from the original Region 24 800 MHz plan under Docket 93-131 |  |  |  |  |
| County Name | FCC Channel | Mobile Frequency | Base Frequency | Channel Comments |
| Adair | 79 | 852.05 | 807.05 | Original allotment 1993 Group T |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group T |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group T |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group T |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group T |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group T |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group T |
| Andrew | 31 | 851.4 | 806.4 | Original allotment 1993 Group O |
|  | 34 | 851.4375 | 806.4375 | Original allotment 1993 Group O Coordinate use with Brown Co KS |
|  | 49 | 851.65 | 806.65 | Original allotment 1993 Group O |
| Atchison | 61 | 851.8 | 806.8 | Original allotment 1993 Group S |
|  | 101 | 852.325 | 807.325 | Original allotment 1993 Group S |
|  | 117 | 852.55 | 807.55 | Original allotment 1993 Group S |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group S |
|  | 138 | 852.8125 | 807.8125 | Original allotment 1993 Group S |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group S |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group S |
| Audrain | 23 | 851.3 | 806.3 | Original allotment 1993 Group J |
|  | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group J |
|  | 60 | 851.7875 | 806.7875 | Original allotment 1993 Group J |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group J |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group J |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group J |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group J |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group J |
|  | 199 | 853.6 | 808.6 | Original allotment 1993 Group J |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group J |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group J Reg 13 written concurrence required |
| Barry | 23 | 851.3 | 806.3 | Original allotment 1993 Group K |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K |
| Barton | 19 | 851.25 | 806.25 | Original allotment 1993 Group H |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group H |
|  | 73 | 851.95 | 806.95 | Original allotment 1993 Group H |
|  | 86 | 852.1375 | 807.1375 | Original allotment 1993 Group H |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group H |
|  | 112 | 852.4625 | 807.4625 | Original allotment 1993 Group H |
|  | 114 | 852.4875 | 807.4875 | Original allotment 1993 Group H |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group H |
|  | 126 | 852.6625 | 807.6625 | Original allotment 1993 Group H |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group H |
|  | 227 | 853.95 | 808.95 | Original allotment 1993 Group H |
| Bates | 31 | 851.4 | 806.4 | Original allotment 1993 Group N |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group N |
|  | 67 | 851.875 | 806.875 | Original allotment 1993 Group N |
|  | 142 | 852.8625 | 807.8625 | Original allotment 1993 Group N |
|  | 144 | 852.8875 | 807.8875 | Original allotment 1993 Group N |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group N |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group N |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group N |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group N |
| Benton | 31 | 851.4 | 806.4 | Original allotment 1993 Group N |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group N |
|  | 67 | 851.875 | 806.875 | Original allotment 1993 Group N |
|  | 142 | 852.8625 | 807.8625 | Original allotment 1993 Group N |
|  | 144 | 852.8875 | 807.8875 | Original allotment 1993 Group N |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group N |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group N |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group N |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group N |
| Bollinger | 19 | 851.25 | 806.25 | Original allotment 1993 Group I |
|  | 54 | 851.7125 | 806.7125 | Original allotment 1993 Group I |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group I Reg 13 Reject |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Group I |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group I Reg 13 Reject |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group I |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Group I |
|  | 166 | 853.1875 | 808.1875 | Original allotment 1993 Group I |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Group I |
|  | 200 | 853.6125 | 808.6125 | Original allotment 1993 Group I |
|  | 205 | 853.675 | 808.675 | Original allotment 1993 Group I Reg 13 Reject |
|  | 226 | 853.9375 | 808.9375 | Original allotment 1993 Group I |
| Boone | 12 | 851.1625 | 806.1625 | Original allotment 1993 Group E |
|  | 51 | 851.675 | 806.675 | Original allotment 1993 Group E |
|  | 53 | 851.7 | 806.7 | Original allotment 1993 Group E |
|  | 70 | 851.9125 | 806.9125 | Original allotment 1993 Group E |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Group E |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Group E |
|  | 85 | 852.125 | 807.125 | Original allotment 1993 Group E |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group E |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Group E |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Group E |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group E |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group E Reg 13 Reject |
|  | 127 | 852.675 | 807.675 | Original allotment 1993 Group E |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group E |
|  | 184 | 853.4125 | 808.4125 | Original allotment 1993 Group E |
|  | 204 | 853.6625 | 808.6625 | Original allotment 1993 Group E |
|  | 224 | 853.9125 | 808.9125 | Original allotment 1993 Group E |
| Buchanan | 31 | 851.4 | 806.4 | Original allotment 1993: Group O Applicant licensed channels outside the approved Region 24 plan. Callsigns WPOZ590 and WQDD838 license issued Sept, 99 |
|  | 34 | 851.4375 | 806.4375 | Original allotment 1993: Group O Applicant licensed channels outside the approved Region 24 plan. Callsigns WPOZ590 and WQDD838 license issued Sept, 99 |
|  | 49 | 851.65 | 806.65 | Original allotment 1993: Group O Applicant licensed channels outside the approved Region 24 plan, Callsigns WPOZ590 and WQDD838 license issued Sept, 99 |
| Butler | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group P |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group P |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group P |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group P |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group P |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group P |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group P |
|  | 157 | 853.075 | 808.075 | Original allotment 1993 Group P |
|  | 215 | 853.8 | 808.8 | Original allotment 1993 Group P Reg 13 Reject |
| Caldwell | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M |
| Callaway | 12 | 851.1625 | 806.1625 | Original allotment 1993 Group E Reg 13 Reject |
|  | 51 | 851.675 | 806.675 | Original allotment 1993 Group E Reg 13 written concurrence required |
|  | 53 | 851.7 | 806.7 | Original allotment 1993 Group E |
|  | 70 | 851.9125 | 806.9125 | Original allotment 1993 Group E |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Group E |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Group E |
|  | 85 | 852.125 | 807.125 | Original allotment 1993 Group E Reg 13 written concurrence required |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group E Reg 13 Reject |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Group E |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Group E |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group E |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group E Reg 13 Reject |
|  | 127 | 852.675 | 807.675 | Original allotment 1993 Group E |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group E Reg 13 Reject |
|  | 184 | 853.4125 | 808.4125 | Original allotment 1993 Group E |
|  | 204 | 853.6625 | 808.6625 | Original allotment 1993 Group E |
|  | 224 | 853.9125 | 808.9125 | Original allotment 1993 Group E |
| Camden | 2 | 851.0375 | 806.0375 | addition Oct 2008 |
|  | 5 | 851.075 | 806.075 | Original allotment 1993 Group C |
|  | 7 | 851.1 | 806.1 | Original allotment 1993 Group C |
|  | 9 | 851.125 | 806.125 | Original allotment 1993 Group C |
|  | 24 | 851.3125 | 806.3125 | addition Oct 2008 |
|  | 47 | 851.625 | 806.625 | addition Oct 2008 |
|  | 65 | 851.85 | 806.85 | Original allotment 1993 Group C |
|  | 71 | 851.925 | 806.925 | addition Oct 2008 |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group C |
|  | 95 | 852.25 | 807.25 | addition Oct 2008 |
|  | 119 | 852.575 | 807.575 | addition Oct 2008 |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group C |
|  | 141 | 852.85 | 807.85 | addition Oct 2008 |
|  | 164 | 853.1625 | 808.1625 | addition Oct 2008 |
|  | 181 | 853.375 | 808.375 | Original allotment 1993 Group C |
|  | 188 | 853.4625 | 808.4625 | addition Oct 2008 |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group C |
|  | 198 | 853.5875 | 808.5875 | Original allotment 1993 Group C |
|  | 212 | 853.7625 | 808.7625 | addition Oct 2008 |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group C |
|  | 218 | 853.8375 | 808.8375 | Original allotment 1993 Group C |
|  | 220 | 853.8625 | 808.8625 | Original allotment 1993 Group C |
|  | 229 | 853.975 | 808.975 | addition Oct 2008 |
| Cape Girardeau | 19 | 851.25 | 806.25 | Original allotment 1993 Group I Reg 13 written concurrence required |
|  | 54 | 851.7125 | 806.7125 | Original allotment 1993 Group I Reg 13 written concurrence required |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group I Reg 13 Reject |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Group I Reg 13 Reject |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group I Reg 13 Reject |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group I Reg 13 Reject |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Group I |
|  | 166 | 853.1875 | 808.1875 | Original allotment 1993 Group I |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Group I Reg 13 written concurrence required |
|  | 200 | 853.6125 | 808.6125 | Original allotment 1993 Group I |
|  | 205 | 853.675 | 808.675 | Original allotment 1993 Group I Reg 13 Reject |
|  | 226 | 853.9375 | 808.9375 | Original allotment 1993 Group I |
| Carroll | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M |
| Carter | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group P |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group P |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group P |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group P |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group P |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group P |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group P |
|  | 157 | 853.075 | 808.075 | Original allotment 1993 Group P |
|  | 215 | 853.8 | 808.8 | Original allotment 1993 Group P |
| Cass | 31 | 851.4 | 806.4 | Original allotment 1993 Group N |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group N |
|  | 67 | 851.875 | 806.875 | Original allotment 1993 Group N |
|  | 142 | 852.8625 | 807.8625 | Original allotment 1993 Group N |
|  | 144 | 852.8875 | 807.8875 | Original allotment 1993 Group N |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group N |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group N |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group N |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group N |
| Cedar | 31 | 851.4 | 806.4 | Original allotment 1993 Group N |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group N |
|  | 67 | 851.875 | 806.875 | Original allotment 1993 Group N |
|  | 142 | 852.8625 | 807.8625 | Original allotment 1993 Group N |
|  | 144 | 852.8875 | 807.8875 | Original allotment 1993 Group N |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group N |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group N |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group N |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group N |
| Chariton | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M |
| Christian | 23 | 851.3 | 806.3 | Original allotment 1993 Group K |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K |
| Clark | 79 | 852.05 | 807.05 | Original allotment 1993 Group T |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group T Reg 13 written concurrence required |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group T |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group T Reg 13 Reject |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group T |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group T |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group T |
| Clay | 18 | 851.2375 | 806.2375 | Original allotment 1993 Group G |
|  | 20 | 851.2625 | 806.2625 | Original allotment 1993 Group G |
|  | 38 | 851.4875 | 806.4875 | Original allotment 1993 Group G |
|  | 56 | 851.7375 | 806.7375 | Original allotment 1993 Group G |
|  | 58 | 851.7625 | 806.7625 | Original allotment 1993 Group G |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group G |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Group G |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Group G |
|  | 76 | 851.9875 | 806.9875 | Original allotment 1993 Group G |
|  | 79 | 852.05 | 807.05 | Original allotment 1993 Group G |
|  | 81 | 852.075 | 807.075 | Original allotment 1993 Group G |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Group G |
|  | 86 | 852.1375 | 807.1375 | Original allotment 1993 Group G |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group G |
|  | 93 | 852.225 | 807.225 | Original allotment 1993 Group G |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group G |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group G |
|  | 103 | 852.35 | 807.35 | Original allotment 1993 Group G |
|  | 106 | 852.3875 | 807.3875 | Original allotment 1993 Group G |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Group G |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Group G |
|  | 112 | 852.4625 | 807.4625 | Original allotment 1993 Group G |
|  | 114 | 852.4875 | 807.4875 | Original allotment 1993 Group G |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group G |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group G |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group G |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group G |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group G |
|  | 126 | 852.6625 | 807.6625 | Original allotment 1993 Group G |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Group G |
|  | 132 | 852.7375 | 807.7375 | Original allotment 1993 Group G |
|  | 143 | 852.875 | 807.875 | Original allotment 1993 Group G |
|  | 152 | 852.9875 | 807.9875 | Original allotment 1993 Group G |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group G |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group G |
|  | 163 | 853.15 | 808.15 | Original allotment 1993 Group G |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Group G |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group G |
|  | 190 | 853.4875 | 808.4875 | Original allotment 1993 Group G |
|  | 198 | 853.5875 | 808.5875 | Original allotment 1993 Group G |
|  | 210 | 853.7375 | 808.7375 | Original allotment 1993 Group G |
|  | 227 | 853.95 | 808.95 | Original allotment 1993 Group G |
|  | 229 | 853.975 | 808.975 | Original allotment 1993 Group G |
| Clinton | 31 | 851.4 | 806.4 | Original allotment 1993 Group O |
|  | 34 | 851.4375 | 806.4375 | Original allotment 1993 Group O |
|  | 49 | 851.65 | 806.65 | Original allotment 1993 Group O |
| Cole | 8 | 851.1125 | 806.1125 | Addition October 2008 |
|  | 12 | 851.1625 | 806.1625 | Original allotment 1993 Group E |
|  | 19 | 851.25 | 806.25 | Addition October 2008 |
|  | 34 | 851.4375 | 806.4375 | Addition October 2008 |
|  | 35 | 851.45 | 806.45 | Addition October 2008 |
|  | 40 | 851.5375 | 806.5375 | Addition October 2008 |
|  | 51 | 851.675 | 806.675 | Original allotment 1993 Group E |
|  | 53 | 851.7 | 806.7 | Original allotment 1993 Group E |
|  | 55 | 851.725 | 806.725 | Addition October 2008 |
|  | 56 | 851.7375 | 806.7375 | Addition October 2008 |
|  | 59 | 851.775 | 806.775 | Addition October 2008 |
|  | 70 | 851.9125 | 806.9125 | Original allotment 1993 Group E |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Group E |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Group E |
|  | 81 | 852.075 | 807.075 | Addition October 2008 |
|  | 83 | 852.1 | 807.1 | Addition October 2008 |
|  | 85 | 852.125 | 807.125 | Original allotment 1993 Group E |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group E |
|  | 93 | 852.225 | 807.225 | Addition October 2008 |
|  | 103 | 852.35 | 807.35 | Addition October 2008 |
|  | 105 | 852.375 | 807.375 | Addition October 2008 |
|  | 107 | 852.4 | 807.4 | Addition October 2008 |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Group E |
|  | 109 | 852.425 | 807.425 | Addition October 2008 |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Group E |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group E |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group E |
|  | 126 | 852.6625 | 807.6625 | Addition October 2008 |
|  | 127 | 852.675 | 807.675 | Original allotment 1993 Group E |
|  | 128 | 852.6875 | 807.6875 | Addition October 2008 |
|  | 131 | 852.725 | 807.725 | Addition October 2008 |
|  | 146 | 852.9125 | 807.9125 | Addition October 2008 |
|  | 147 | 852.925 | 807.925 | Addition October 2008 |
|  | 148 | 852.9375 | 807.9375 | Addition October 2008 |
|  | 152 | 852.9875 | 807.9875 | Addition October 2008 |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group E |
|  | 161 | 853.125 | 808.125 | Addition October 2008 |
|  | 163 | 853.15 | 808.15 | Addition October 2008 |
|  | 170 | 853.2375 | 808.2375 | Addition October 2008 |
|  | 184 | 853.4125 | 808.4125 | Original allotment 1993 Group E |
|  | 185 | 853.425 | 808.425 | Addition October 2008 |
|  | 200 | 853.6125 | 808.6125 | Addition October 2008 |
|  | 204 | 853.6625 | 808.6625 | Original allotment 1993 Group E |
|  | 209 | 853.725 | 808.725 | Addition October 2008 |
|  | 217 | 853.825 | 808.825 | Addition October 2008 |
|  | 224 | 853.9125 | 808.9125 | Original allotment 1993 Group E |
|  | 230 | 853.9875 | 808.9875 | Addition October 2008 |
| Cooper | 4 | 851.0625 | 806.0625 | Addition October 2008 |
|  | 12 | 851.1625 | 806.1625 | Original allotment 1993 Group E |
|  | 30 | 851.3875 | 806.3875 | Addition October 2008 |
|  | 50 | 851.6625 | 806.6625 | Addition October 2008 |
|  | 51 | 851.675 | 806.675 | Original allotment 1993 Group E |
|  | 53 | 851.7 | 806.7 | Original allotment 1993 Group E |
|  | 70 | 851.9125 | 806.9125 | Original allotment 1993 Group E |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Group E |
|  | 73 | 851.95 | 806.95 | Addition October 2008 |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Group E |
|  | 85 | 852.125 | 807.125 | Original allotment 1993 Group E |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group E |
|  | 94 | 852.2375 | 807.2375 | Addition October 2008 |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Group E |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Group E |
|  | 115 | 852.5125 | 807.5125 | Addition October 2008 |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group E |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group E |
|  | 127 | 852.675 | 807.675 | Original allotment 1993 Group E |
|  | 135 | 852.775 | 807.775 | Addition October 2008 |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group E |
|  | 165 | 853.175 | 808.175 | Addition October 2008 |
|  | 184 | 853.4125 | 808.4125 | Original allotment 1993 Group E |
|  | 186 | 853.4375 | 808.4375 | Addition October 2008 |
|  | 204 | 853.6625 | 808.6625 | Original allotment 1993 Group E |
|  | 206 | 853.6875 | 808.6875 | Addition October 2008 |
|  | 224 | 853.9125 | 808.9125 | Original allotment 1993 Group E |
|  | 228 | 853.9625 | 808.9625 | Addition October 2008 |
| Crawford | 15 | 851.2 | 806.2 | Original allotment 1993 Group F Reg 13 Reject |
|  | 17 | 851.225 | 806.225 | Original allotment 1993 Group F Reg 13 Reject |
|  | 22 | 851.2875 | 806.2875 | Original allotment 1993 Group F |
|  | 49 | 851.65 | 806.65 | Original allotment 1993 Group F |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group F |
|  | 132 | 852.7375 | 807.7375 | Original allotment 1993 Group F Reg 13 written concurrence required |
|  | 149 | 852.95 | 807.95 | Original allotment 1993 Group F |
|  | 159 | 853.1 | 808.1 | Original allotment 1993 Group F |
|  | 168 | 853.2125 | 808.2125 | Original allotment 1993 Group F |
| Dade | 23 | 851.3 | 806.3 | Original allotment 1993 Group K |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K |
| Dallas | 23 | 851.3 | 806.3 | Original allotment 1993 Group K |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K |
| Daviess | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M Per Region 16 coordinate use of this channel with Atchison Co KS |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M Per Region 16 requests coordinate use with Brown Co KS |
| De Kalb | 31 | 851.4 | 806.4 | Original allotment 1993 Group O |
|  | 34 | 851.4375 | 806.4375 | Original allotment 1993 Group O |
|  | 49 | 851.65 | 806.65 | Original allotment 1993 Group O |
| Dent | 15 | 851.2 | 806.2 | Original allotment 1993 Group F |
|  | 17 | 851.225 | 806.225 | Original allotment 1993 Group F |
|  | 22 | 851.2875 | 806.2875 | Original allotment 1993 Group F |
|  | 49 | 851.65 | 806.65 | Original allotment 1993 Group F |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group F |
|  | 132 | 852.7375 | 807.7375 | Original allotment 1993 Group F |
|  | 149 | 852.95 | 807.95 | Original allotment 1993 Group F |
|  | 159 | 853.1 | 808.1 | Original allotment 1993 Group F |
|  | 168 | 853.2125 | 808.2125 | Original allotment 1993 Group F |
| Douglas | 2 | 851.0375 | 806.0375 | Original allotment 1993 Group A |
|  | 11 | 851.15 | 806.15 | Original allotment 1993 Group A |
|  | 30 | 851.3875 | 806.3875 | Original allotment 1993 Group A |
|  | 32 | 851.4125 | 806.4125 | Original allotment 1993 Group A |
|  | 106 | 852.3875 | 807.3875 | Original allotment 1993 Group A |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group A |
|  | 141 | 852.85 | 807.85 | Original allotment 1993 Group A |
|  | 143 | 852.875 | 807.875 | Original allotment 1993 Group A |
|  | 179 | 853.35 | 808.35 | Original allotment 1993 Group A |
|  | 183 | 853.4 | 808.4 | Original allotment 1993 Group A |
| Dunklin | 51 | 851.675 | 806.675 | Original allotment 1993 Group Q |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group Q |
|  | 93 | 852.225 | 807.225 | Original allotment 1993 Group Q |
|  | 131 | 852.725 | 807.725 | Original allotment 1993 Group Q |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group Q |
|  | 151 | 852.975 | 807.975 | Original allotment 1993 Group Q |
|  | 163 | 853.15 | 808.15 | Original allotment 1993 Group Q Coor with Reg 39 |
|  | 185 | 853.425 | 808.425 | Original allotment 1993 Group Q |
|  | 207 | 853.7 | 808.7 | Original allotment 1993 Group Q Coor with Reg 39 |
|  | 219 | 853.85 | 808.85 | Original allotment 1993 Group Q |
| Franklin | 4 | 851.0625 | 806.0625 | Addition October 2008 |
|  | 6 | 851.0875 | 806.0875 | Addition October 2008 |
|  | 9 | 851.125 | 806.125 | Addition October 2008 |
|  | 14 | 851.1875 | 806.1875 | Addition October 2008 |
|  | 16 | 851.2125 | 806.2125 | Addition October 2008 |
|  | 18 | 851.2375 | 806.2375 | Addition October 2008 |
|  | 24 | 851.3125 | 806.3125 | Addition October 2008 |
|  | 31 | 851.4 | 806.4 | Addition October 2008 |
|  | 37 | 851.475 | 806.475 | Addition October 2008 |
|  | 41 | 851.55 | 806.55 | Addition October 2008 |
|  | 52 | 851.6875 | 806.6875 | Addition October 2008 |
|  | 54 | 851.7125 | 806.7125 | Addition October 2008 |
|  | 62 | 851.8125 | 806.8125 | Addition October 2008 |
|  | 64 | 851.8375 | 806.8375 | Addition October 2008 |
|  | 65 | 851.85 | 806.85 | Addition October 2008 |
|  | 72 | 851.9375 | 806.9375 | Addition October 2008 |
|  | 73 | 851.95 | 806.95 | Addition October 2008 |
|  | 81 | 852.075 | 807.075 | Addition October 2008 |
|  | 83 | 852.1 | 807.1 | Addition October 2008 |
|  | 87 | 852.15 | 807.15 | Addition October 2008 |
|  | 99 | 852.3 | 807.3 | Addition October 2008 |
|  | 105 | 852.375 | 807.375 | Addition October 2008 |
|  | 106 | 852.3875 | 807.3875 | Addition October 2008 |
|  | 126 | 852.6625 | 807.6625 | Addition October 2008 |
|  | 130 | 852.7125 | 807.7125 | Addition October 2008 |
|  | 132 | 852.7375 | 807.7375 | Addition October 2008 |
|  | 134 | 852.7625 | 807.7625 | Addition October 2008 |
|  | 137 | 852.8 | 807.8 | Addition October 2008 |
|  | 145 | 852.9 | 807.9 | Addition October 2008 |
|  | 146 | 852.9125 | 807.9125 | Addition October 2008 |
|  | 161 | 853.125 | 808.125 | Addition October 2008 |
|  | 163 | 853.15 | 808.15 | Addition October 2008 |
|  | 166 | 853.1875 | 808.1875 | Addition October 2008 |
|  | 167 | 853.2 | 808.2 | Addition October 2008 |
|  | 169 | 853.225 | 808.225 | Addition October 2008 |
|  | 173 | 853.275 | 808.275 | Addition October 2008 |
|  | 176 | 853.3125 | 808.3125 | Addition October 2008 |
|  | 188 | 853.4625 | 808.4625 | Addition October 2008 |
|  | 201 | 853.625 | 808.625 | Addition October 2008 |
|  | 203 | 853.65 | 808.65 | Addition October 2008 |
|  | 204 | 853.6625 | 808.6625 | Addition October 2008 |
|  | 216 | 853.8125 | 808.8125 | Addition October 2008 |
|  | 218 | 853.8375 | 808.8375 | Addition October 2008 |
|  | 220 | 853.8625 | 808.8625 | Addition October 2008 |
|  | 228 | 853.9625 | 808.9625 | Addition October 2008 |
| Gasconade | 15 | 851.2 | 806.2 | Original allotment 1993 Group F Reg 13 Reject |
|  | 17 | 851.225 | 806.225 | Original allotment 1993 Group F Reg 13 Reject |
|  | 22 | 851.2875 | 806.2875 | Original allotment 1993 Group F Reg 13 written concurrence required |
|  | 49 | 851.65 | 806.65 | Original allotment 1993 Group F |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group F |
|  | 132 | 852.7375 | 807.7375 | Original allotment 1993 Group F Reg 13 Reject |
|  | 149 | 852.95 | 807.95 | Original allotment 1993 Group F |
|  | 159 | 853.1 | 808.1 | Original allotment 1993 Group F |
|  | 168 | 853.2125 | 808.2125 | Original allotment 1993 Group F Reg 13 Reject |
| Gentry | 61 | 851.8 | 806.8 | Original allotment 1993 Group S |
|  | 101 | 852.325 | 807.325 | Original allotment 1993 Group S |
|  | 117 | 852.55 | 807.55 | Original allotment 1993 Group S |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group S |
|  | 138 | 852.8125 | 807.8125 | Original allotment 1993 Group S |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group S |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group S |
| Greene | 23 | 851.3 | 806.3 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K channels 601, 639, 677, 715 and 753 (CALL and ITAC Channels) used under callsigns WPKP318 and WPMH498 |
| Grundy | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M |
| Harrison | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M |
| Henry | 31 | 851.4 | 806.4 | Original allotment 1993 Group N |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group N |
|  | 67 | 851.875 | 806.875 | Original allotment 1993 Group N |
|  | 142 | 852.8625 | 807.8625 | Original allotment 1993 Group N |
|  | 144 | 852.8875 | 807.8875 | Original allotment 1993 Group N |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group N |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group N |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group N |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group N |
| Hickory | 31 | 851.4 | 806.4 | Original allotment 1993 Group N |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group N |
|  | 67 | 851.875 | 806.875 | Original allotment 1993 Group N |
|  | 142 | 852.8625 | 807.8625 | Original allotment 1993 Group N |
|  | 144 | 852.8875 | 807.8875 | Original allotment 1993 Group N |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group N |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group N |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group N |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group N |
| Holt | 61 | 851.8 | 806.8 | Original allotment 1993 Group S |
|  | 101 | 852.325 | 807.325 | Original allotment 1993 Group S |
|  | 117 | 852.55 | 807.55 | Original allotment 1993 Group S |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group S |
|  | 138 | 852.8125 | 807.8125 | Original allotment 1993 Group S |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group S |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group S |
| Howard | 12 | 851.1625 | 806.1625 | Original allotment 1993 Group E |
|  | 51 | 851.675 | 806.675 | Original allotment 1993 Group E |
|  | 53 | 851.7 | 806.7 | Original allotment 1993 Group E |
|  | 70 | 851.9125 | 806.9125 | Original allotment 1993 Group E |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Group E |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Group E |
|  | 85 | 852.125 | 807.125 | Original allotment 1993 Group E |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group E |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Group E |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Group E |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group E |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group E Reg 13 written concurrence required |
|  | 127 | 852.675 | 807.675 | Original allotment 1993 Group E |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group E |
|  | 184 | 853.4125 | 808.4125 | Original allotment 1993 Group E |
|  | 204 | 853.6625 | 808.6625 | Original allotment 1993 Group E |
|  | 224 | 853.9125 | 808.9125 | Original allotment 1993 Group E |
| Howell | 2 | 851.0375 | 806.0375 | Original allotment 1993 Group A |
|  | 11 | 851.15 | 806.15 | Original allotment 1993 Group A |
|  | 30 | 851.3875 | 806.3875 | Original allotment 1993 Group A |
|  | 32 | 851.4125 | 806.4125 | Original allotment 1993 Group A |
|  | 106 | 852.3875 | 807.3875 | Original allotment 1993 Group A |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group A |
|  | 141 | 852.85 | 807.85 | Original allotment 1993 Group A |
|  | 143 | 852.875 | 807.875 | Original allotment 1993 Group A |
|  | 179 | 853.35 | 808.35 | Original allotment 1993 Group A |
|  | 183 | 853.4 | 808.4 | Original allotment 1993 Group A |
| Iron | 19 | 851.25 | 806.25 | Original allotment 1993 Group I Reg 13 written concurrence required |
|  | 54 | 851.7125 | 806.7125 | Original allotment 1993 Group I |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group I Reg 13 written concurrence required |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Group I |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group I Reg 13 Reject |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group I |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Group I |
|  | 166 | 853.1875 | 808.1875 | Original allotment 1993 Group I |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Group I |
|  | 200 | 853.6125 | 808.6125 | Original allotment 1993 Group I |
|  | 205 | 853.675 | 808.675 | Original allotment 1993 Group I Reg 13 Reject |
|  | 226 | 853.9375 | 808.9375 | Original allotment 1993 Group I |
| Jackson | 18 | 851.2375 | 806.2375 | Group G Platte/Jackson County available channels after Region 16 objections Region 16 requires coordination of this channel with use in Jefferson County KS |
|  | 20 | 851.2625 | 806.2625 | Group G Platte/Jackson County available channels after Region 16 objections Region 16 requires coordination of this channel with use in Jefferson County KS |
|  | 38 | 851.4875 | 806.4875 | Group G Platte/Jackson County available channels after Region 16 objections: Region 16 requires coordination with use of this channel with Jefferson County KS |
|  | 56 | 851.7375 | 806.7375 | Group G Platte/Jackson County available channels after Region 16 objections: Region 16 requires coordination of this frequency with use in Wyandotte County KS |
|  | 58 | 851.7625 | 806.7625 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 63 | 851.825 | 806.825 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 72 | 851.9375 | 806.9375 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 74 | 851.9625 | 806.9625 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 76 | 851.9875 | 806.9875 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 79 | 852.05 | 807.05 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 81 | 852.075 | 807.075 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 83 | 852.1 | 807.1 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 86 | 852.1375 | 807.1375 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 89 | 852.175 | 807.175 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 93 | 852.225 | 807.225 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 95 | 852.25 | 807.25 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 100 | 852.3125 | 807.3125 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 103 | 852.35 | 807.35 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 106 | 852.3875 | 807.3875 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 108 | 852.4125 | 807.4125 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 110 | 852.4375 | 807.4375 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 112 | 852.4625 | 807.4625 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 114 | 852.4875 | 807.4875 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 116 | 852.5375 | 807.5375 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 118 | 852.5625 | 807.5625 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 120 | 852.5875 | 807.5875 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 122 | 852.6125 | 807.6125 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 124 | 852.6375 | 807.6375 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 126 | 852.6625 | 807.6625 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 128 | 852.6875 | 807.6875 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 132 | 852.7375 | 807.7375 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: Region 16 requires |
|  |  |  |  | coordination of use of this channel with |
|  |  |  |  | Leavenworth Co KS |
|  | 143 | 852.875 | 807.875 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 152 | 852.9875 | 807.9875 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: Region 16 requires |
|  |  |  |  | coordinatred use of this channel with use |
|  |  |  |  | in Shawnee Co KS |
|  | 154 | 853.0375 | 808.0375 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 156 | 853.0625 | 808.0625 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 163 | 853.15 | 808.15 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 170 | 853.2375 | 808.2375 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: Region 16 requires |
|  |  |  |  | coordinated use of this channel with Shawnee |
|  |  |  |  | County KS |
|  | 176 | 853.3125 | 808.3125 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 190 | 853.4875 | 808.4875 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: Region 16 requires |
|  |  |  |  | coordinated use of this channel with Franklin |
|  |  |  |  | Co KS |
|  | 198 | 853.5875 | 808.5875 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: Region 16 requires |
|  |  |  |  | coordinated use of this channel with Kansas |
|  |  |  |  | Statewide |
|  | 210 | 853.7375 | 808.7375 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: Region 16 requires |
|  |  |  |  | coordinated use of this channel with Franklin |
|  |  |  |  | Co KS |
|  | 227 | 853.95 | 808.95 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
|  | 229 | 853.975 | 808.975 | Group G Platte/Jackson County available channels |
|  |  |  |  | after Region 16 objections: 658, 663, 672, |
|  |  |  |  | 674, 676, 679, 681, 686, 689, 693, 695, |
|  |  |  |  | 700, 703, 706, 708, 710, 712, 714, 716, |
|  |  |  |  | 718, 720, 722, 724, 726, 728, 743, 754, |
|  |  |  |  | 756, 763, 776, 827, 829 |
| Jasper | 19 | 851.25 | 806.25 | Original allotment 1993 Group H |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group H |
|  | 73 | 851.95 | 806.95 | Original allotment 1993 Group H |
|  | 86 | 852.1375 | 807.1375 | Original allotment 1993 Group H |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group H |
|  | 112 | 852.4625 | 807.4625 | Original allotment 1993 Group H |
|  | 114 | 852.4875 | 807.4875 | Original allotment 1993 Group H |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group H |
|  | 126 | 852.6625 | 807.6625 | Original allotment 1993 Group H |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group H |
|  | 227 | 853.95 | 808.95 | Original allotment 1993 Group H |
| Jefferson | 3 | 851.05 | 806.05 | Added January 2009 |
|  | 11 | 851.15 | 806.15 | Added January 2009 |
|  | 19 | 851.25 | 806.25 | Added January 2009 |
|  | 26 | 851.3375 | 806.3375 | Added January 2009 |
|  | 28 | 851.3625 | 806.3625 | Added January 2009 |
|  | 33 | 851.425 | 806.425 | Added January 2009 |
|  | 38 | 851.4875 | 806.4875 | Added January 2009 |
|  | 44 | 851.5875 | 806.5875 | Added January 2009 |
|  | 46 | 851.6125 | 806.6125 | Added January 2009 |
|  | 51 | 851.675 | 806.675 | Added January 2009 |
|  | 55 | 851.725 | 806.725 | Added January 2009 |
|  | 63 | 851.825 | 806.825 | Added January 2009 |
|  | 68 | 851.8875 | 806.8875 | Added January 2009 |
|  | 76 | 851.9875 | 806.9875 | Added January 2009 |
|  | 84 | 852.1125 | 807.1125 | Added January 2009 |
|  | 86 | 852.1375 | 807.1375 | Added January 2009 |
|  | 100 | 852.3125 | 807.3125 | Added January 2009 |
|  | 107 | 852.4 | 807.4 | Added January 2009 |
|  | 116 | 852.5375 | 807.5375 | Added January 2009 |
|  | 135 | 852.775 | 807.775 | Added January 2009 |
|  | 142 | 852.8625 | 807.8625 | Added January 2009 |
|  | 156 | 853.0625 | 808.0625 | Added January 2009 |
|  | 158 | 853.0875 | 808.0875 | Added January 2009 |
|  | 170 | 853.2375 | 808.2375 | Added January 2009 |
|  | 182 | 853.3875 | 808.3875 | Added January 2009 |
|  | 189 | 853.475 | 808.475 | Added January 2009 |
|  | 191 | 853.5 | 808.5 | Added January 2009 |
|  | 193 | 853.525 | 808.525 | Added January 2009 |
|  | 195 | 853.55 | 808.55 | Added January 2009 |
|  | 197 | 853.575 | 808.575 | Added January 2009 |
|  | 215 | 853.8 | 808.8 | Added January 2009 |
|  | 225 | 853.925 | 808.925 | Added January 2009 |
|  | 227 | 853.95 | 808.95 | Added January 2009 |
|  | 229 | 853.975 | 808.975 | Added January 2009 |
| Johnson | 3 | 851.05 | 806.05 | Original allotment 1993 Group B |
|  | 16 | 851.2125 | 806.2125 | Original allotment 1993 Group B |
|  | 25 | 851.325 | 806.325 | Original allotment 1993 Group B Per Region 16 coordinate use of this channel with Bourbon Co KS |
|  | 27 | 851.35 | 806.35 | Original allotment 1993 Group B |
|  | 33 | 851.425 | 806.425 | Original allotment 1993 Group B |
|  | 36 | 851.4625 | 806.4625 | Original allotment 1993 Group B |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group B |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group B Per Region 16 coordinate use of this channel with Douglas Co KS |
|  | 150 | 852.9625 | 807.9625 | Original allotment 1993 Group B |
| Knox | 79 | 852.05 | 807.05 | Original allotment 1993 Group T |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group T |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group T |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group T |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group T |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group T |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group T |
| Laclede | 5 | 851.075 | 806.075 | Original allotment 1993 Group C |
|  | 7 | 851.1 | 806.1 | Original allotment 1993 Group C |
|  | 9 | 851.125 | 806.125 | Original allotment 1993 Group C |
|  | 65 | 851.85 | 806.85 | Original allotment 1993 Group C |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group C |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group C |
|  | 181 | 853.375 | 808.375 | Original allotment 1993 Group C |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group C |
|  | 198 | 853.5875 | 808.5875 | Original allotment 1993 Group C |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group C |
|  | 218 | 853.8375 | 808.8375 | Original allotment 1993 Group C |
|  | 220 | 853.8625 | 808.8625 | Original allotment 1993 Group C |
| Lafayette | 3 | 851.05 | 806.05 | Original allotment 1993 Group B |
|  | 16 | 851.2125 | 806.2125 | Original allotment 1993 Group B |
|  | 25 | 851.325 | 806.325 | Original allotment 1993 Group B |
|  | 27 | 851.35 | 806.35 | Original allotment 1993 Group B |
|  | 33 | 851.425 | 806.425 | Original allotment 1993 Group B |
|  | 36 | 851.4625 | 806.4625 | Original allotment 1993 Group B |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group B |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group B Per region 16 coordinate use of this channel with Douglas CO KS |
|  | 150 | 852.9625 | 807.9625 | Original allotment 1993 Group B |
| Lawrence | 23 | 851.3 | 806.3 | Original allotment 1993 Group K |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K |
| Lewis | 79 | 852.05 | 807.05 | Original allotment 1993 Group T |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group T Reg 13 Reject |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group T |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group T Reg 13 Reject |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group T |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group T |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group T |
| Lincoln | 10 | 851.1375 | 806.1375 | Original allotment 1993 Group D AVAILABLE Can be coordinated with Region 13 |
|  | 23 | 851.3 | 806.3 | New allotment ? Could be used in Lincoln Co |
|  | 79 | 852.05 | 807.05 | Original allotment 1993 Group D Reg 13 Reject |
|  | 87 | 852.15 | 807.15 | Original allotment 1993 Group D AVAILABLE Can be coordinated with Region 13 |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group D Reg 13 Reject |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group D Reg 13 Reject |
|  | 157 | 853.075 | 808.075 | Original allotment 1993 Group D AVAILABLE Region 13 Can be coordinated for use in south part of county |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group D Reg 13 Reject |
|  | 164 | 853.1625 | 808.1625 | Original allotment 1993 Group D AVAILABLE R 13 written concurrence required can be coordinated with Region 13 |
| Linn | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M |
| Livingston | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M |
| Macon | 23 | 851.3 | 806.3 | Original allotment 1993 Group J |
|  | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group J |
|  | 60 | 851.7875 | 806.7875 | Original allotment 1993 Group J |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group J |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group J |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group J |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group J |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group J |
|  | 199 | 853.6 | 808.6 | Original allotment 1993 Group J |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group J |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group J |
| Madison | 19 | 851.25 | 806.25 | Original allotment 1993 Group I Reg 13 written concurrence required |
|  | 54 | 851.7125 | 806.7125 | Original allotment 1993 Group I |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group I Reg 13 Reject |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Group I Reg 13 written concurrence required |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group I Reg 13 Reject |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group I Reg 13 written concurrence required |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Group I |
|  | 166 | 853.1875 | 808.1875 | Original allotment 1993 Group I |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Group I |
|  | 200 | 853.6125 | 808.6125 | Original allotment 1993 Group I |
|  | 205 | 853.675 | 808.675 | Original allotment 1993 Group I Reg 13 Reject |
|  | 226 | 853.9375 | 808.9375 | Original allotment 1993 Group I |
| Maries | 15 | 851.2 | 806.2 | Original allotment 1993 Group F |
|  | 17 | 851.225 | 806.225 | Original allotment 1993 Group F |
|  | 22 | 851.2875 | 806.2875 | Original allotment 1993 Group F |
|  | 49 | 851.65 | 806.65 | Original allotment 1993 Group F |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group F |
|  | 132 | 852.7375 | 807.7375 | Original allotment 1993 Group F |
|  | 149 | 852.95 | 807.95 | Original allotment 1993 Group F |
|  | 159 | 853.1 | 808.1 | Original allotment 1993 Group F |
|  | 168 | 853.2125 | 808.2125 | Original allotment 1993 Group F |
| Marion | 23 | 851.3 | 806.3 | Original allotment 1993 Group J Reg 13 Reject |
|  | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group J |
|  | 60 | 851.7875 | 806.7875 | Original allotment 1993 Group J |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group J |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group J |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group J |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group J |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group J |
|  | 199 | 853.6 | 808.6 | Original allotment 1993 Group J |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group J |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group J Reg 13 Reject |
| McDonald | 19 | 851.25 | 806.25 | Original allotment 1993 Group H |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group H |
|  | 73 | 851.95 | 806.95 | Original allotment 1993 Group H |
|  | 86 | 852.1375 | 807.1375 | Original allotment 1993 Group H |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group H |
|  | 112 | 852.4625 | 807.4625 | Original allotment 1993 Group H |
|  | 114 | 852.4875 | 807.4875 | Original allotment 1993 Group H |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group H |
|  | 126 | 852.6625 | 807.6625 | Original allotment 1993 Group H |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group H |
|  | 227 | 853.95 | 808.95 | Original allotment 1993 Group H |
| Mercer | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M |
| Miller | 5 | 851.075 | 806.075 | Original allotment 1993 Group C |
|  | 7 | 851.1 | 806.1 | Original allotment 1993 Group C |
|  | 9 | 851.125 | 806.125 | Original allotment 1993 Group C |
|  | 13 | 851.175 | 806.175 | Addition Oct 2008 |
|  | 29 | 851.375 | 806.375 | Addition Oct 2008 |
|  | 37 | 851.475 | 806.475 | Addition Oct 2008 |
|  | 54 | 851.7125 | 806.7125 | Addition Oct 2008 |
|  | 62 | 851.8125 | 806.8125 | Addition Oct 2008 |
|  | 65 | 851.85 | 806.85 | Original allotment 1993 Group C |
|  | 75 | 851.975 | 806.975 | Addition Oct 2008 |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group C |
|  | 91 | 852.2 | 807.2 | Addition Oct 2008 |
|  | 101 | 852.325 | 807.325 | Addition Oct 2008 |
|  | 114 | 852.4875 | 807.4875 | Addition Oct 2008 |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group C |
|  | 121 | 852.6 | 807.6 | Addition Oct 2008 |
|  | 138 | 852.8125 | 807.8125 | Addition Oct 2008 |
|  | 169 | 853.225 | 808.225 | Addition Oct 2008 |
|  | 181 | 853.375 | 808.375 | Original allotment 1993 Group C |
|  | 190 | 853.4875 | 808.4875 | Addition Oct 2008 |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group C |
|  | 198 | 853.5875 | 808.5875 | Original allotment 1993 Group C |
|  | 207 | 853.7 | 808.7 | Addition Oct 2008 |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group C |
|  | 218 | 853.8375 | 808.8375 | Original allotment 1993 Group C |
|  | 220 | 853.8625 | 808.8625 | Original allotment 1993 Group C |
|  | 227 | 853.95 | 808.95 | Addition Oct 2008 |
| Mississippi | 51 | 851.675 | 806.675 | Original allotment 1993 Group Q Reg 13 Reject |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group Q Reg 13 Reject |
|  | 93 | 852.225 | 807.225 | Original allotment 1993 Group Q |
|  | 131 | 852.725 | 807.725 | Original allotment 1993 Group Q |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group Q |
|  | 151 | 852.975 | 807.975 | Original allotment 1993 Group Q |
|  | 163 | 853.15 | 808.15 | Original allotment 1993 Group Q Reg 13 Reject |
|  | 185 | 853.425 | 808.425 | Original allotment 1993 Group Q |
|  | 207 | 853.7 | 808.7 | Original allotment 1993 Group Q |
|  | 219 | 853.85 | 808.85 | Original allotment 1993 Group Q |
| Moniteau | 11 | 851.15 | 806.15 | addition Oct 2008 |
|  | 12 | 851.1625 | 806.1625 | Original allotment 1993 Group E |
|  | 26 | 851.3375 | 806.3375 | addition Oct 2008 |
|  | 46 | 851.6125 | 806.6125 | addition Oct 2008 |
|  | 51 | 851.675 | 806.675 | Original allotment 1993 Group E |
|  | 53 | 851.7 | 806.7 | Original allotment 1993 Group E |
|  | 69 | 851.9 | 806.9 | addition Oct 2008 |
|  | 70 | 851.9125 | 806.9125 | Original allotment 1993 Group E |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Group E |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Group E |
|  | 85 | 852.125 | 807.125 | Original allotment 1993 Group E |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group E |
|  | 90 | 852.1875 | 807.1875 | addition Oct 2008 |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Group E |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Group E |
|  | 111 | 852.45 | 807.45 | addition Oct 2008 |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group E |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group E |
|  | 127 | 852.675 | 807.675 | Original allotment 1993 Group E |
|  | 133 | 852.75 | 807.75 | addition Oct 2008 |
|  | 153 | 853.0125 | 808.0125 | addition Oct 2008 |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group E |
|  | 173 | 853.275 | 808.275 | addition Oct 2008 |
|  | 184 | 853.4125 | 808.4125 | Original allotment 1993 Group E |
|  | 193 | 853.525 | 808.525 | addition Oct 2008 |
|  | 204 | 853.6625 | 808.6625 | Original allotment 1993 Group E |
|  | 215 | 853.8 | 808.8 | addition Oct 2008 |
|  | 224 | 853.9125 | 808.9125 | Original allotment 1993 Group E |
| Monroe | 23 | 851.3 | 806.3 | Original allotment 1993 Group J |
|  | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group J |
|  | 60 | 851.7875 | 806.7875 | Original allotment 1993 Group J |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group J |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group J |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group J |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group J |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group J |
|  | 199 | 853.6 | 808.6 | Original allotment 1993 Group J |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group J |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group J Reg 13 written concurrence required |
| Montgomery | 10 | 851.1375 | 806.1375 | Original allotment 1993 Group D |
|  | 23 | 851.3 | 806.3 | New allotment, can be coordinated with Region 13 |
|  | 79 | 852.05 | 807.05 | Original allotment 1993 Group D Reg 13 Reject |
|  | 87 | 852.15 | 807.15 | Original allotment 1993 Group D AVAILABLE Can be coordinated with Region 13 |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group D AVAILABLE Can be coordinated with Region 13 |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group D AVAILABLE Can be coordinated with Region 13 |
|  | 157 | 853.075 | 808.075 | Original allotment 1993 Group D AVAILABLE Can be coordinated with Region 13 |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group D AVAILABLE R 13 Reject Can be coordinated with Region 13 |
|  | 164 | 853.1625 | 808.1625 | Original allotment 1993 Group D AVAILABLE Can be coordinated with Region 13 |
| Morgan | 5 | 851.075 | 806.075 | Original allotment 1993 Group C |
|  | 6 | 851.0875 | 806.0875 | addition Oct 2008 |
|  | 7 | 851.1 | 806.1 | Original allotment 1993 Group C |
|  | 9 | 851.125 | 806.125 | Original allotment 1993 Group C |
|  | 14 | 851.1875 | 806.1875 | addition Oct 2008 |
|  | 21 | 851.275 | 806.275 | addition Oct 2008 |
|  | 39 | 851.5125 | 806.5125 | addition Oct 2008 |
|  | 48 | 851.6375 | 806.6375 | addition Oct 2008 |
|  | 57 | 851.75 | 806.75 | addition Oct 2008 |
|  | 64 | 851.8375 | 806.8375 | addition Oct 2008 |
|  | 65 | 851.85 | 806.85 | Original allotment 1993 Group C |
|  | 77 | 852.0125 | 807.0125 | addition Oct 2008 |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group C |
|  | 88 | 852.1625 | 807.1625 | addition Oct 2008 |
|  | 97 | 852.275 | 807.275 | addition Oct 2008 |
|  | 117 | 852.55 | 807.55 | addition Oct 2008 |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group C |
|  | 134 | 852.7625 | 807.7625 | addition Oct 2008 |
|  | 166 | 853.1875 | 808.1875 | addition Oct 2008 |
|  | 181 | 853.375 | 808.375 | Original allotment 1993 Group C |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group C |
|  | 195 | 853.55 | 808.55 | addition Oct 2008 |
|  | 198 | 853.5875 | 808.5875 | Original allotment 1993 Group C |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group C |
|  | 218 | 853.8375 | 808.8375 | Original allotment 1993 Group C |
|  | 219 | 853.85 | 808.85 | addition Oct 2008 |
|  | 220 | 853.8625 | 808.8625 | Original allotment 1993 Group C |
| New Madrid | 51 | 851.675 | 806.675 | Original allotment 1993 Group Q Reg 13 written concurrence required |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group Q Reg 13 written concurrence required |
|  | 93 | 852.225 | 807.225 | Original allotment 1993 Group Q |
|  | 131 | 852.725 | 807.725 | Original allotment 1993 Group Q |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group Q |
|  | 151 | 852.975 | 807.975 | Original allotment 1993 Group Q |
|  | 163 | 853.15 | 808.15 | Original allotment 1993 Group Q Reg 13 written concurrence required |
|  | 185 | 853.425 | 808.425 | Original allotment 1993 Group Q |
|  | 207 | 853.7 | 808.7 | Original allotment 1993 Group Q |
|  | 219 | 853.85 | 808.85 | Original allotment 1993 Group Q |
| Newton | 19 | 851.25 | 806.25 | Original allotment 1993 Group H Per Region 16 coordinate use of this channel with Crawford Co KS |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group H |
|  | 73 | 851.95 | 806.95 | Original allotment 1993 Group H |
|  | 86 | 852.1375 | 807.1375 | Original allotment 1993 Group H |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group H |
|  | 112 | 852.4625 | 807.4625 | Original allotment 1993 Group H |
|  | 114 | 852.4875 | 807.4875 | Original allotment 1993 Group H |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group H |
|  | 126 | 852.6625 | 807.6625 | Original allotment 1993 Group H |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group H |
|  | 227 | 853.95 | 808.95 | Original allotment 1993 Group H |
| Nodaway | 61 | 851.8 | 806.8 | Original allotment 1993 Group S |
|  | 101 | 852.325 | 807.325 | Original allotment 1993 Group S |
|  | 117 | 852.55 | 807.55 | Original allotment 1993 Group S |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group S |
|  | 138 | 852.8125 | 807.8125 | Original allotment 1993 Group S |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group S |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group S |
| Oregon | 2 | 851.0375 | 806.0375 | Original allotment 1993 Group A |
|  | 11 | 851.15 | 806.15 | Original allotment 1993 Group A |
|  | 30 | 851.3875 | 806.3875 | Original allotment 1993 Group A |
|  | 32 | 851.4125 | 806.4125 | Original allotment 1993 Group A |
|  | 106 | 852.3875 | 807.3875 | Original allotment 1993 Group A |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group A |
|  | 141 | 852.85 | 807.85 | Original allotment 1993 Group A |
|  | 143 | 852.875 | 807.875 | Original allotment 1993 Group A |
|  | 179 | 853.35 | 808.35 | Original allotment 1993 Group A |
|  | 183 | 853.4 | 808.4 | Original allotment 1993 Group A |
| Osage | 12 | 851.1625 | 806.1625 | Original allotment 1993 Group E Reg 13 written concurrence required |
|  | 51 | 851.675 | 806.675 | Original allotment 1993 Group E |
|  | 53 | 851.7 | 806.7 | Original allotment 1993 Group E |
|  | 70 | 851.9125 | 806.9125 | Original allotment 1993 Group E |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Group E |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Group E |
|  | 85 | 852.125 | 807.125 | Original allotment 1993 Group E |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group E Reg 13 written concurrence required |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Group E |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Group E |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group E |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group E Reg 13 written concurrence required |
|  | 127 | 852.675 | 807.675 | Original allotment 1993 Group E |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group E Reg 13 written concurrence required |
|  | 184 | 853.4125 | 808.4125 | Original allotment 1993 Group E |
|  | 204 | 853.6625 | 808.6625 | Original allotment 1993 Group E |
|  | 224 | 853.9125 | 808.9125 | Original allotment 1993 Group E |
| Ozark | 2 | 851.0375 | 806.0375 | Original allotment 1993 Group A |
|  | 11 | 851.15 | 806.15 | Original allotment 1993 Group A |
|  | 30 | 851.3875 | 806.3875 | Original allotment 1993 Group A |
|  | 32 | 851.4125 | 806.4125 | Original allotment 1993 Group A |
|  | 106 | 852.3875 | 807.3875 | Original allotment 1993 Group A |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group A |
|  | 141 | 852.85 | 807.85 | Original allotment 1993 Group A |
|  | 143 | 852.875 | 807.875 | Original allotment 1993 Group A |
|  | 179 | 853.35 | 808.35 | Original allotment 1993 Group A |
|  | 183 | 853.4 | 808.4 | Original allotment 1993 Group A |
| Pemiscot | 51 | 851.675 | 806.675 | Original allotment 1993 Group Q |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group Q |
|  | 93 | 852.225 | 807.225 | Original allotment 1993 Group Q |
|  | 131 | 852.725 | 807.725 | Original allotment 1993 Group Q |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group Q |
|  | 151 | 852.975 | 807.975 | Original allotment 1993 Group Q |
|  | 163 | 853.15 | 808.15 | Original allotment 1993 Group Q Coor with Reg 39 |
|  | 185 | 853.425 | 808.425 | Original allotment 1993 Group Q |
|  | 207 | 853.7 | 808.7 | Original allotment 1993 Group Q Coor with Reg 39 |
|  | 219 | 853.85 | 808.85 | Original allotment 1993 Group Q |
| Perry | 19 | 851.25 | 806.25 | Original allotment 1993 Group I Reg 13 Reject |
|  | 54 | 851.7125 | 806.7125 | Original allotment 1993 Group I Reg 13 Reject |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group I Reg 13 Reject |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Group I Reg 13 Reject |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group I Reg 13 Reject |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group I Reg 13 Reject |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Group I |
|  | 166 | 853.1875 | 808.1875 | Original allotment 1993 Group I |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Group I Reg 13 written concurrence required |
|  | 200 | 853.6125 | 808.6125 | Original allotment 1993 Group I |
|  | 205 | 853.675 | 808.675 | Original allotment 1993 Group I Reg 13 Reject |
|  | 226 | 853.9375 | 808.9375 | Original allotment 1993 Group I |
| Pettis | 3 | 851.05 | 806.05 | Original allotment 1993 Group B |
|  | 16 | 851.2125 | 806.2125 | Original allotment 1993 Group B |
|  | 25 | 851.325 | 806.325 | Original allotment 1993 Group B |
|  | 27 | 851.35 | 806.35 | Original allotment 1993 Group B |
|  | 33 | 851.425 | 806.425 | Original allotment 1993 Group B |
|  | 36 | 851.4625 | 806.4625 | Original allotment 1993 Group B |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group B |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group B |
|  | 150 | 852.9625 | 807.9625 | Original allotment 1993 Group B |
| Phelps | 15 | 851.2 | 806.2 | Original allotment 1993 Group F |
|  | 17 | 851.225 | 806.225 | Original allotment 1993 Group F |
|  | 22 | 851.2875 | 806.2875 | Original allotment 1993 Group F |
|  | 49 | 851.65 | 806.65 | Original allotment 1993 Group F |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group F |
|  | 132 | 852.7375 | 807.7375 | Original allotment 1993 Group F |
|  | 149 | 852.95 | 807.95 | Original allotment 1993 Group F |
|  | 159 | 853.1 | 808.1 | Original allotment 1993 Group F |
|  | 168 | 853.2125 | 808.2125 | Original allotment 1993 Group F |
| Pike | 23 | 851.3 | 806.3 | Original allotment 1993 Group J Coor w R 13 adj |
|  | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group J 13 Reject |
|  | 60 | 851.7875 | 806.7875 | Original allotment 1993 Group J |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group J 13 Reject |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group J 13 Reject |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group J |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group J |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group J 13 Reject |
|  | 199 | 853.6 | 808.6 | Original allotment 1993 Group J 13 Reject |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group J 13 Reject |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group J 13 Reject |
| Platte | 18 | 851.2375 | 806.2375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 20 | 851.2625 | 806.2625 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 38 | 851.4875 | 806.4875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 56 | 851.7375 | 806.7375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 58 | 851.7625 | 806.7625 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 76 | 851.9875 | 806.9875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 79 | 852.05 | 807.05 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 81 | 852.075 | 807.075 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 86 | 852.1375 | 807.1375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 93 | 852.225 | 807.225 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 103 | 852.35 | 807.35 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 106 | 852.3875 | 807.3875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 112 | 852.4625 | 807.4625 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 114 | 852.4875 | 807.4875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 126 | 852.6625 | 807.6625 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 132 | 852.7375 | 807.7375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 143 | 852.875 | 807.875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 152 | 852.9875 | 807.9875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 163 | 853.15 | 808.15 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 190 | 853.4875 | 808.4875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 198 | 853.5875 | 808.5875 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 210 | 853.7375 | 808.7375 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 227 | 853.95 | 808.95 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 229 | 853.975 | 808.975 | Original allotment 1993 Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
| Polk | 23 | 851.3 | 806.3 | Original allotment 1993 Group K |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K |
| Pulaski | 5 | 851.075 | 806.075 | Original allotment 1993 Group C |
|  | 7 | 851.1 | 806.1 | Original allotment 1993 Group C |
|  | 9 | 851.125 | 806.125 | Original allotment 1993 Group C |
|  | 18 | 851.2375 | 806.2375 | Addition Oct 2008 |
|  | 38 | 851.4875 | 806.4875 | Addition Oct 2008 |
|  | 44 | 851.5875 | 806.5875 | Addition Oct 2008 |
|  | 52 | 851.6875 | 806.6875 | Addition Oct 2008 |
|  | 65 | 851.85 | 806.85 | Original allotment 1993 Group C |
|  | 66 | 851.8625 | 806.8625 | Addition Oct 2008 |
|  | 76 | 851.9875 | 806.9875 | Addition Oct 2008 |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group C |
|  | 86 | 852.1375 | 807.1375 | Addition Oct 2008 |
|  | 96 | 852.2625 | 807.2625 | Addition Oct 2008 |
|  | 113 | 852.475 | 807.475 | Addition Oct 2008 |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group C |
|  | 123 | 852.625 | 807.625 | Addition Oct 2008 |
|  | 129 | 852.7 | 807.7 | Addition Oct 2008 |
|  | 145 | 852.9 | 807.9 | Addition Oct 2008 |
|  | 160 | 853.1125 | 808.1125 | Addition Oct 2008 |
|  | 167 | 853.2 | 808.2 | Addition Oct 2008 |
|  | 181 | 853.375 | 808.375 | Original allotment 1993 Group C |
|  | 182 | 853.3875 | 808.3875 | Addition Oct 2008 |
|  | 187 | 853.45 | 808.45 | Addition Oct 2008 |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group C |
|  | 198 | 853.5875 | 808.5875 | Original allotment 1993 Group C |
|  | 203 | 853.65 | 808.65 | Addition Oct 2008 |
|  | 211 | 853.75 | 808.75 | Addition Oct 2008 |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group C |
|  | 218 | 853.8375 | 808.8375 | Original allotment 1993 Group C |
|  | 220 | 853.8625 | 808.8625 | Original allotment 1993 Group C |
|  | 225 | 853.925 | 808.925 | Addition Oct 2008 |
| Putnam | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M Reg 13 written concurrence required |
| Ralls | 23 | 851.3 | 806.3 | Original allotment 1993 Group J 13 Reject |
|  | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group J |
|  | 60 | 851.7875 | 806.7875 | Original allotment 1993 Group J |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group J |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group J |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group J |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group J |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group J Reg 13 written concurrence required |
|  | 199 | 853.6 | 808.6 | Original allotment 1993 Group J |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group J Reg 13 written concurrence required |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group J 13 Reject |
| Randolph | 23 | 851.3 | 806.3 | Original allotment 1993 Group J |
|  | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group J |
|  | 60 | 851.7875 | 806.7875 | Original allotment 1993 Group J |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group J |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group J |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group J |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group J |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group J |
|  | 199 | 853.6 | 808.6 | Original allotment 1993 Group J |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group J |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group J |
| Ray | 18 | 851.2375 | 806.2375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 20 | 851.2625 | 806.2625 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 38 | 851.4875 | 806.4875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 56 | 851.7375 | 806.7375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 58 | 851.7625 | 806.7625 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 72 | 851.9375 | 806.9375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 74 | 851.9625 | 806.9625 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 76 | 851.9875 | 806.9875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 79 | 852.05 | 807.05 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 81 | 852.075 | 807.075 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 86 | 852.1375 | 807.1375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 93 | 852.225 | 807.225 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 103 | 852.35 | 807.35 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 106 | 852.3875 | 807.3875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 108 | 852.4125 | 807.4125 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 110 | 852.4375 | 807.4375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 112 | 852.4625 | 807.4625 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 114 | 852.4875 | 807.4875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 126 | 852.6625 | 807.6625 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 132 | 852.7375 | 807.7375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 143 | 852.875 | 807.875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 152 | 852.9875 | 807.9875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 163 | 853.15 | 808.15 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 190 | 853.4875 | 808.4875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 198 | 853.5875 | 808.5875 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 210 | 853.7375 | 808.7375 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 227 | 853.95 | 808.95 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
|  | 229 | 853.975 | 808.975 | Original allotment 1993 Group G Platte/Jackson/Clay/Ray County available |
|  |  |  |  | channels after Region 16 objections: 658, |
|  |  |  |  | 663, 672, 674, 676, 679, 681, 686, 689, |
|  |  |  |  | 693, 695, 700, 703, 706, 708, 710, 712, |
|  |  |  |  | 714, 716, 718, 720, 722, 724, 726, 728, |
|  |  |  |  | 743, 754, 756, 763, 776, 827, 829 |
| Reynolds | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group P Reg 13 written concurrence required |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group P |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group P |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group P |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group P |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group P |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group P |
|  | 157 | 853.075 | 808.075 | Original allotment 1993 Group P |
|  | 215 | 853.8 | 808.8 | Original allotment 1993 Group P |
| Ripley | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group P |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group P |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group P |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group P |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group P |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group P |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group P |
|  | 157 | 853.075 | 808.075 | Original allotment 1993 Group P |
|  | 215 | 853.8 | 808.8 | Original allotment 1993 Group P |
| Saline | 3 | 851.05 | 806.05 | Original allotment 1993 Group B |
|  | 16 | 851.2125 | 806.2125 | Original allotment 1993 Group B |
|  | 25 | 851.325 | 806.325 | Original allotment 1993 Group B |
|  | 27 | 851.35 | 806.35 | Original allotment 1993 Group B |
|  | 33 | 851.425 | 806.425 | Original allotment 1993 Group B |
|  | 36 | 851.4625 | 806.4625 | Original allotment 1993 Group B |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group B |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group B |
|  | 150 | 852.9625 | 807.9625 | Original allotment 1993 Group B |
| Schuyler | 79 | 852.05 | 807.05 | Original allotment 1993 Group T |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group T |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group T |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group T |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group T |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group T |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group T |
| Scotland | 79 | 852.05 | 807.05 | Original allotment 1993 Group T |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group T |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group T |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group T |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group T |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group T |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group T |
| Scott | 51 | 851.675 | 806.675 | Original allotment 1993 Group Q 13 Reject |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group Q 13 Reject |
|  | 93 | 852.225 | 807.225 | Original allotment 1993 Group Q |
|  | 131 | 852.725 | 807.725 | Original allotment 1993 Group Q Reg 13 written concurrence required |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group Q |
|  | 151 | 852.975 | 807.975 | Original allotment 1993 Group Q |
|  | 163 | 853.15 | 808.15 | Original allotment 1993 Group Q 13 Reject |
|  | 185 | 853.425 | 808.425 | Original allotment 1993 Group Q Coor w R 13 adj |
|  | 207 | 853.7 | 808.7 | Original allotment 1993 Group Q |
|  | 219 | 853.85 | 808.85 | Original allotment 1993 Group Q |
| Shannon | 2 | 851.0375 | 806.0375 | Original allotment 1993 Group A |
|  | 11 | 851.15 | 806.15 | Original allotment 1993 Group A |
|  | 30 | 851.3875 | 806.3875 | Original allotment 1993 Group A |
|  | 32 | 851.4125 | 806.4125 | Original allotment 1993 Group A |
|  | 106 | 852.3875 | 807.3875 | Original allotment 1993 Group A |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group A |
|  | 141 | 852.85 | 807.85 | Original allotment 1993 Group A |
|  | 143 | 852.875 | 807.875 | Original allotment 1993 Group A |
|  | 179 | 853.35 | 808.35 | Original allotment 1993 Group A |
|  | 183 | 853.4 | 808.4 | Original allotment 1993 Group A |
| Shelby | 23 | 851.3 | 806.3 | Original allotment 1993 Group J |
|  | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group J |
|  | 60 | 851.7875 | 806.7875 | Original allotment 1993 Group J |
|  | 82 | 852.0875 | 807.0875 | Original allotment 1993 Group J |
|  | 120 | 852.5875 | 807.5875 | Original allotment 1993 Group J |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group J |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group J |
|  | 194 | 853.5375 | 808.5375 | Original allotment 1993 Group J |
|  | 199 | 853.6 | 808.6 | Original allotment 1993 Group J |
|  | 214 | 853.7875 | 808.7875 | Original allotment 1993 Group J |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group J Reg 13 written concurrence required |
| St. Charles | 21 | 851.275 | 806.275 | St Charles County is a part of the North Zone within the St Louis UASI in Region 24. The North Zone consists of St Louis County, St Charles County and the City of St Louis. The allotments in these areas are interchangeable in the Region 24 800 MHz plan, conditioned on coordination and approval of their adjacent region. |
|  | 23 | 851.3 | 806.3 |  |
|  | 40 | 851.5375 | 806.5375 |  |
|  | 50 | 851.6625 | 806.6625 |  |
|  | 79 | 852.05 | 807.05 |  |
|  | 94 | 852.2375 | 807.2375 |  |
|  | 95 | 852.25 | 807.25 |  |
|  | 97 | 852.275 | 807.275 |  |
|  | 108 | 852.4125 | 807.4125 |  |
|  | 110 | 852.4375 | 807.4375 |  |
|  | 117 | 852.55 | 807.55 |  |
|  | 122 | 852.6125 | 807.6125 |  |
|  | 124 | 852.6375 | 807.6375 |  |
|  | 140 | 852.8375 | 807.8375 |  |
|  | 147 | 852.925 | 807.925 |  |
|  | 148 | 852.9375 | 807.9375 |  |
|  | 154 | 853.0375 | 808.0375 |  |
|  | 155 | 853.05 | 808.05 |  |
|  | 157 | 853.075 | 808.075 |  |
|  | 160 | 853.1125 | 808.1125 |  |
|  | 164 | 853.1625 | 808.1625 |  |
|  | 171 | 853.25 | 808.25 |  |
|  | 172 | 853.2625 | 808.2625 |  |
|  | 175 | 853.3 | 808.3 |  |
|  | 185 | 853.425 | 808.425 |  |
|  | 187 | 853.45 | 808.45 |  |
|  | 190 | 853.4875 | 808.4875 |  |
|  | 206 | 853.6875 | 808.6875 |  |
|  | 222 | 853.8875 | 808.8875 |  |
|  | 224 | 853.9125 | 808.9125 |  |
| St. Clair | 31 | 851.4 | 806.4 | Original allotment 1993 Group N |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group N |
|  | 67 | 851.875 | 806.875 | Original allotment 1993 Group N |
|  | 142 | 852.8625 | 807.8625 | Original allotment 1993 Group N |
|  | 144 | 852.8875 | 807.8875 | Original allotment 1993 Group N |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group N |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group N |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group N |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group N |
| St. Francois | 19 | 851.25 | 806.25 | Original allotment 1993 Group I 13 Reject |
|  | 54 | 851.7125 | 806.7125 | Original allotment 1993 Group I |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group I 13 Reject |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Group I 13 Reject |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group I 13 Reject |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group I 13 Reject |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Group I |
|  | 166 | 853.1875 | 808.1875 | Original allotment 1993 Group I |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Group I |
|  | 200 | 853.6125 | 808.6125 | Original allotment 1993 Group I |
|  | 205 | 853.675 | 808.675 | Original allotment 1993 Group I 13 Reject |
|  | 226 | 853.9375 | 808.9375 | Original allotment 1993 Group I |
|  |  |  |  | St Louis City is a part of the North Zone within the St Louis UASI in Region 24. The North Zone consists of St Louis County, St Charles County and the City of St Louis. The allotments in these areas are interchangeable in the Region 24 800 MHz plan, conditioned on coordination and approval of their adjacent region. |
| St. Louis City | 10 | 851.1375 | 806.1375 | NEW To be considered adj |
|  | 14 | 851.1875 | 806.1875 | NEW To be considered adj |
|  | 43 | 851.575 | 806.575 | NEW To be considered adj |
|  | 49 | 851.65 | 806.65 | NEW To be considered adj |
|  | 52 | 851.6875 | 806.6875 | NEW to be considered adj |
|  | 54 | 851.7125 | 806.7125 | NEW to be considered adj |
|  | 58 | 851.7625 | 806.7625 | Original allotment 1993 13 Reject Group R Consider adj REJECTED Per Region 13 objections, channels permitted for use in St Louis City are 712, 751, 779, 788, 828 |
|  | 60 | 851.7875 | 806.7875 | NEW To be considered adj |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 13 Reject Group R REJECTED Per Region 13 objections, channels permitted for use in St Louis City are:712, 751, 779, 788, 828 |
|  | 92 | 852.2125 | 807.2125 | Original allotment 1993 13 Reject Group R REJECTED Per Region 13 objections, |
|  |  |  |  | channels permitted for use in St Louis City |
|  |  |  |  | are:712, 751, 779, 788, 828 |
|  | 99 | 852.3 | 807.3 | Original allotment 1993 13 Reject Group R REJECTED Per Region 13 objections, |
|  |  |  |  | channels permitted for use in St Louis City |
|  |  |  |  | are:712, 751, 779, 788, 828 |
|  | 102 | 852.3375 | 807.3375 | Added Feb 2009 METRO BSADA use in St Louis City |
|  | 112 | 852.4625 | 807.4625 | Added Feb 2009 METRO BSDA use in St Louis City |
|  |  |  |  | permitted for use in St Louis City are:712, |
|  |  |  |  | 751, 779, 788, 828 |
|  | 114 | 852.4875 | 807.4875 | Added Feb 2009 METRO BSDA use in St Louis City |
|  | 151 | 852.975 | 807.975 | Original allotment 1993 Coor w R 13 adj Group R CONCURRENCE REQUIRED Per Region |
|  |  |  |  | 13 objections, channels permitted for use |
|  |  |  |  | in St Louis City are:712, 751, 779, 788, |
|  |  |  |  | 828 |
|  | 179 | 853.35 | 808.35 | Original allotment 1993 Group R Per Region 13 objections, channels |
|  |  |  |  | permitted for use in St Louis City are:712, |
|  |  |  |  | 751, 779, 788, 828 |
|  | 188 | 853.4625 | 808.4625 | Original allotment 1993 Group R Per Region 13 objections, channels |
|  |  |  |  | permitted for use in St Louis City are:712, |
|  |  |  |  | 751, 779, 788, 828 |
|  | 202 | 853.6375 | 808.6375 | Original allotment 1993 13 Reject Group R REJECTED Per Region 13 objections, |
|  |  |  |  | channels permitted for use in St Louis City |
|  |  |  |  | are:712, 751, 779, 788, 828 |
|  | 208 | 853.7125 | 808.7125 | Original allotment 1993 13 Reject Group R REJECTED Per Region 13 objections, |
|  |  |  |  | channels permitted for use in St Louis City |
|  |  |  |  | are:712, 751, 779, 788, 828 |
|  | 210 | 853.7375 | 808.7375 | Original allotment 1993 13 Reject Group R REJECTED Per Region 13 objections, |
|  |  |  |  | channels permitted for use in St Louis City |
|  |  |  |  | are:712, 751, 779, 788, 828 |
|  | 213 | 853.775 | 808.775 | Original allotment 1993 13 Reject Group R REJECTED Per Region 13 objections, |
|  |  |  |  | channels permitted for use in St Louis City |
|  |  |  |  | are:712, 751, 779, 788, 828 |
|  | 219 | 853.85 | 808.85 | Original allotment 1993 13 Reject Group R REJECTED Per Region 13 objections, |
|  |  |  |  | channels permitted for use in St Louis City |
|  |  |  |  | are:712, 751, 779, 788, 828 |
|  | 228 | 853.9625 | 808.9625 | Original allotment 1993 Group R Per Region 13 objections, channe |
| St. Louis County |  |  |  | St Louis County is a part of the North Zone within the St Louis UASI in Region 24. The North Zone consists of St Louis County, St Charles County and the City of St Louis. The allotments in these areas are interchangeable in the Region 24 800 MHz plan, conditioned on coordination and approval of their adjacent region. |
|  | 2 | 851.0375 | 806.0375 | Added January 2009 |
|  | 7 | 851.1 | 806.1 | Added January 2009 |
|  | 11 | 851.15 | 806.15 | Added January 2009 |
|  | 20 | 851.2625 | 806.2625 | Added January 2009 |
|  | 21 | 851.275 | 806.275 | Added January 2009 |
|  | 22 | 851.2875 | 806.2875 | Added January 2009 |
|  | 28 | 851.3625 | 806.3625 | Added January 2009 |
|  | 32 | 851.4125 | 806.4125 | Added January 2009 |
|  | 34 | 851.4375 | 806.4375 | Added January 2009 |
|  | 35 | 851.45 | 806.45 | Added January 2009 |
|  | 36 | 851.4625 | 806.4625 | Added January 2009 |
|  | 43 | 851.575 | 806.575 | Added January 2009 |
|  | 44 | 851.5875 | 806.5875 | Added January 2009 |
|  | 45 | 851.6 | 806.6 | Added January 2009 |
|  | 47 | 851.625 | 806.625 | Added January 2009 |
|  | 49 | 851.65 | 806.65 | Added January 2009 |
|  | 56 | 851.7375 | 806.7375 | Added January 2009 |
|  | 68 | 851.8875 | 806.8875 | Added January 2009 |
|  | 70 | 851.9125 | 806.9125 | Added January 2009 |
|  | 71 | 851.925 | 806.925 | Added January 2009 |
|  | 74 | 851.9625 | 806.9625 | Added January 2009 |
|  | 75 | 851.975 | 806.975 | Added January 2009 |
|  | 78 | 852.0375 | 807.0375 | Added January 2009 |
|  | 80 | 852.0625 | 807.0625 | Added January 2009 |
|  | 85 | 852.125 | 807.125 | Added January 2009 |
|  | 93 | 852.225 | 807.225 | Added January 2009 |
|  | 94 | 852.2375 | 807.2375 | Added January 2009 |
|  | 96 | 852.2625 | 807.2625 | Added January 2009 |
|  | 98 | 852.2875 | 807.2875 | Added January 2009 |
|  | 109 | 852.425 | 807.425 | Added January 2009 |
|  | 118 | 852.5625 | 807.5625 | Added January 2009 |
|  | 123 | 852.625 | 807.625 | Added January 2009 |
|  | 125 | 852.65 | 807.65 | Added January 2009 |
|  | 127 | 852.675 | 807.675 | Added January 2009 |
|  | 136 | 852.7875 | 807.7875 | Added January 2009 |
|  | 139 | 852.825 | 807.825 | Added January 2009 |
|  | 140 | 852.8375 | 807.8375 | Added January 2009 |
|  | 142 | 852.8625 | 807.8625 | Added January 2009 |
|  | 149 | 852.95 | 807.95 | Added January 2009 |
|  | 154 | 853.0375 | 808.0375 | Added January 2009 |
|  | 159 | 853.1 | 808.1 | Added January 2009 |
|  | 165 | 853.175 | 808.175 | Added January 2009 |
|  | 168 | 853.2125 | 808.2125 | Added January 2009 |
|  | 174 | 853.2875 | 808.2875 | Added January 2009 |
|  | 177 | 853.325 | 808.325 | Added January 2009 |
|  | 182 | 853.3875 | 808.3875 | Added January 2009 |
|  | 184 | 853.4125 | 808.4125 | Added January 2009 |
|  | 186 | 853.4375 | 808.4375 | Added January 2009 |
|  | 190 | 853.4875 | 808.4875 | Added January 2009 |
|  | 192 | 853.5125 | 808.5125 | Added January 2009 |
|  | 194 | 853.5375 | 808.5375 | Added January 2009 |
|  | 196 | 853.5625 | 808.5625 | Added January 2009 |
|  | 198 | 853.5875 | 808.5875 | Added January 2009 |
|  | 200 | 853.6125 | 808.6125 | Added January 2009 |
|  | 205 | 853.675 | 808.675 | Added January 2009 |
|  | 214 | 853.7875 | 808.7875 | Added January 2009 |
|  | 217 | 853.825 | 808.825 | Added January 2009 |
|  | 222 | 853.8875 | 808.8875 | Added January 2009 |
|  | 226 | 853.9375 | 808.9375 | Added January 2009 |
| Ste. Genevieve | 19 | 851.25 | 806.25 | Original allotment 1993 Group I 13 Reject |
|  | 54 | 851.7125 | 806.7125 | Original allotment 1993 Group I 13 Reject |
|  | 57 | 851.75 | 806.75 | Original allotment 1993 Group I 13 Reject |
|  | 83 | 852.1 | 807.1 | Original allotment 1993 Group I 13 Reject |
|  | 95 | 852.25 | 807.25 | Original allotment 1993 Group I 13 Reject |
|  | 125 | 852.65 | 807.65 | Original allotment 1993 Group I 13 Reject |
|  | 128 | 852.6875 | 807.6875 | Original allotment 1993 Group I 13 Reject |
|  | 166 | 853.1875 | 808.1875 | Original allotment 1993 Group I |
|  | 170 | 853.2375 | 808.2375 | Original allotment 1993 Group I |
|  | 200 | 853.6125 | 808.6125 | Original allotment 1993 Group I |
|  | 205 | 853.675 | 808.675 | Original allotment 1993 Group I 13 Reject |
|  | 226 | 853.9375 | 808.9375 | Original allotment 1993 Group I |
| Stoddard | 51 | 851.675 | 806.675 | Original allotment 1993 Group Q Reg 13 written concurrence required |
|  | 89 | 852.175 | 807.175 | Original allotment 1993 Group Q Reg 13 written concurrence required |
|  | 93 | 852.225 | 807.225 | Original allotment 1993 Group Q |
|  | 131 | 852.725 | 807.725 | Original allotment 1993 Group Q Reg 13 written concurrence required |
|  | 133 | 852.75 | 807.75 | Original allotment 1993 Group Q |
|  | 151 | 852.975 | 807.975 | Original allotment 1993 Group Q |
|  | 163 | 853.15 | 808.15 | Original allotment 1993 Group Q Reg 13 written concurrence required |
|  | 185 | 853.425 | 808.425 | Original allotment 1993 Group Q Coor w R 13 adj |
|  | 207 | 853.7 | 808.7 | Original allotment 1993 Group Q |
|  | 219 | 853.85 | 808.85 | Original allotment 1993 Group Q |
| Stone | 23 | 851.3 | 806.3 | Original allotment 1993 Group K |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K |
| Sullivan | 29 | 851.375 | 806.375 | Original allotment 1993 Group M |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group M |
|  | 47 | 851.625 | 806.625 | Original allotment 1993 Group M |
|  | 66 | 851.8625 | 806.8625 | Original allotment 1993 Group M |
|  | 68 | 851.8875 | 806.8875 | Original allotment 1993 Group M |
|  | 140 | 852.8375 | 807.8375 | Original allotment 1993 Group M |
|  | 160 | 853.1125 | 808.1125 | Original allotment 1993 Group M |
|  | 180 | 853.3625 | 808.3625 | Original allotment 1993 Group M |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group M Region 13 written concurrence required |
| Taney | 23 | 851.3 | 806.3 | Original allotment 1993 Group K |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K |
| Texas | 2 | 851.0375 | 806.0375 | Original Allotment 1993 Group A |
|  | 11 | 851.15 | 806.15 | Original Allotment 1993 Group A |
|  | 30 | 851.3875 | 806.3875 | Original Allotment 1993 Group A |
|  | 32 | 851.4125 | 806.4125 | Original Allotment 1993 Group A |
|  | 106 | 852.3875 | 807.3875 | Original Allotment 1993 Group A |
|  | 124 | 852.6375 | 807.6375 | Original Allotment 1993 Group A |
|  | 141 | 852.85 | 807.85 | Original Allotment 1993 Group A |
|  | 143 | 852.875 | 807.875 | Original Allotment 1993 Group A |
|  | 179 | 853.35 | 808.35 | Original Allotment 1993 Group A |
|  | 183 | 853.4 | 808.4 | Original Allotment 1993 Group A |
| Vernon | 31 | 851.4 | 806.4 | Original allotment 1993 Group N |
|  | 45 | 851.6 | 806.6 | Original allotment 1993 Group N |
|  | 67 | 851.875 | 806.875 | Original allotment 1993 Group N |
|  | 142 | 852.8625 | 807.8625 | Original allotment 1993 Group N |
|  | 144 | 852.8875 | 807.8875 | Original allotment 1993 Group N |
|  | 158 | 853.0875 | 808.0875 | Original allotment 1993 Group N |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group N |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group N |
|  | 178 | 853.3375 | 808.3375 | Original allotment 1993 Group N |
| Warren | 10 | 851.1375 | 806.1375 | Original allotment 1993 Group D |
|  | 23 | 851.3 | 806.3 | New Allotment Can be coordinated with Region 13 |
|  | 79 | 852.05 | 807.05 | Original allotment 1993 Group D 13 Reject |
|  | 87 | 852.15 | 807.15 | Original allotment 1993 Group D AVAILABLE Can be coordinated with Region 13 |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group D |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group D AVAILABLE R 13 written concurrence required Can be coordinated with Region 13 |
|  | 157 | 853.075 | 808.075 | Original allotment 1993 Group D AVAILABLE R 13 written concurrence required Can be coordinated with Region 13 |
|  | 162 | 853.1375 | 808.1375 | Original allotment 1993 Group D 13 Reject |
|  | 164 | 853.1625 | 808.1625 | Original allotment 1993 Group D AVAILABLE R 13 written concurrence required can be coordinated with Region 13 |
| Washington | 15 | 851.2 | 806.2 | Original allotment 1993 Group F 13 Reject |
|  | 17 | 851.225 | 806.225 | Original allotment 1993 Group F 13 Reject |
|  | 22 | 851.2875 | 806.2875 | Original allotment 1993 Group F Reg 13 written concurrence required |
|  | 49 | 851.65 | 806.65 | Original allotment 1993 Group F |
|  | 130 | 852.7125 | 807.7125 | Original allotment 1993 Group F |
|  | 132 | 852.7375 | 807.7375 | Original allotment 1993 Group F Reg 13 written concurrence required |
|  | 149 | 852.95 | 807.95 | Original allotment 1993 Group F |
|  | 159 | 853.1 | 808.1 | Original allotment 1993 Group F |
|  | 168 | 853.2125 | 808.2125 | Original allotment 1993 Group F |
| Wayne | 42 | 851.5625 | 806.5625 | Original allotment 1993 Group P Reg 13 written concurrence required |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group P Reg 13 written concurrence required |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group P |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group P |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group P |
|  | 137 | 852.8 | 807.8 | Original allotment 1993 Group P |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group P |
|  | 157 | 853.075 | 808.075 | Original allotment 1993 Group P |
|  | 215 | 853.8 | 808.8 | Original allotment 1993 Group P 13 Reject |
| Webster | 23 | 851.3 | 806.3 | Original allotment 1993 Group K |
|  | 28 | 851.3625 | 806.3625 | Original allotment 1993 Group K |
|  | 41 | 851.55 | 806.55 | Original allotment 1993 Group K |
|  | 43 | 851.575 | 806.575 | Original allotment 1993 Group K |
|  | 61 | 851.8 | 806.8 | Original allotment 1993 Group K |
|  | 63 | 851.825 | 806.825 | Original allotment 1993 Group K |
|  | 78 | 852.0375 | 807.0375 | Original allotment 1993 Group K |
|  | 80 | 852.0625 | 807.0625 | Original allotment 1993 Group K |
|  | 84 | 852.1125 | 807.1125 | Original allotment 1993 Group K |
|  | 98 | 852.2875 | 807.2875 | Original allotment 1993 Group K |
|  | 100 | 852.3125 | 807.3125 | Original allotment 1993 Group K |
|  | 102 | 852.3375 | 807.3375 | Original allotment 1993 Group K |
|  | 104 | 852.3625 | 807.3625 | Original allotment 1993 Group K |
|  | 116 | 852.5375 | 807.5375 | Original allotment 1993 Group K |
|  | 118 | 852.5625 | 807.5625 | Original allotment 1993 Group K |
|  | 122 | 852.6125 | 807.6125 | Original allotment 1993 Group K |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group K |
|  | 139 | 852.825 | 807.825 | Original allotment 1993 Group K |
|  | 154 | 853.0375 | 808.0375 | Original allotment 1993 Group K |
|  | 156 | 853.0625 | 808.0625 | Original allotment 1993 Group K |
|  | 176 | 853.3125 | 808.3125 | Original allotment 1993 Group K |
|  | 196 | 853.5625 | 808.5625 | Original allotment 1993 Group K |
|  | 201 | 853.625 | 808.625 | Original allotment 1993 Group K |
|  | 216 | 853.8125 | 808.8125 | Original allotment 1993 Group K |
| Worth | 61 | 851.8 | 806.8 | Original allotment 1993 Group S |
|  | 101 | 852.325 | 807.325 | Original allotment 1993 Group S |
|  | 117 | 852.55 | 807.55 | Original allotment 1993 Group S |
|  | 136 | 852.7875 | 807.7875 | Original allotment 1993 Group S |
|  | 138 | 852.8125 | 807.8125 | Original allotment 1993 Group S |
|  | 155 | 853.05 | 808.05 | Original allotment 1993 Group S |
|  | 174 | 853.2875 | 808.2875 | Original allotment 1993 Group S |
| Wright | 2 | 851.0375 | 806.0375 | Original allotment 1993 Group A |
|  | 11 | 851.15 | 806.15 | Original allotment 1993 Group A |
|  | 30 | 851.3875 | 806.3875 | Original allotment 1993 Group A |
|  | 32 | 851.4125 | 806.4125 | Original allotment 1993 Group A |
|  | 106 | 852.3875 | 807.3875 | Original allotment 1993 Group A |
|  | 124 | 852.6375 | 807.6375 | Original allotment 1993 Group A |
|  | 141 | 852.85 | 807.85 | Original allotment 1993 Group A |
|  | 143 | 852.875 | 807.875 | Original allotment 1993 Group A |
|  | 179 | 853.35 | 808.35 | Original allotment 1993 Group A |
|  | 183 | 853.4 | 808.4 | Original allotment 1993 Group A |